3/F, 7 KIC, 388 Songhu Rd, Yangpu District, Shanghai，China

SUNMI PAY SDK V2 Development Document v3.2.2

Shanghai Sunmi Tech Co.,Ltd

目录

 1. Revision History	3

 2. Abstract	5

2.1 Introduction	5

2.2 SDK Integration	5

 3. API	6

3.1 SunmiPayKernel SDK Operation Object	6

3.2 Public member variable	7

3.3 Basic operation module	7

3.4 Card operation module	9

3.5 PinPad operation module	15

3.6 Security operation module	16

3.7 EMV operation module	22

 4. Error Code Definition	29

 5. Entity class	35

5.1 EmvTermParamV2 – Terminal parameter entity class	35

5.2 AidV2 -AID entity class	36

5.3 CapkV2 –CAPK entity class	36

5.4 EMVTransDataV2 - Transaction data entity class	36

5.5 EMVCandidateV2 – EMV Application Candidate	37

5.5 PinPadConfigV2 - Pinpad configuration entity class	37

 6. Access permission	38

6.1 Permission location	38

6.2 Permission definition	38

 7. Appendix	39

7.1 Aidl constants class（com.sunmi.pay.hardware.aidl.AidlConstants）	39

7.2 PAN data interception	41

7.3 EMV trans flow chart	43

- 1. Revision History

| Doc Version | Release | Changes | The appropriate SDK version |  |
| --- | --- | --- | --- | --- |
| 3.0.0 | 2017/11/30 | 1st Draft | SunmiPaySDKService1.0.1 |  |
| 3.0.1 | 2018/1/22 | Revise Docs | SunmiPaySDKService1.0.3/SunmiPayHardwareService3.0.44 |  |
| 3.0.2 | 2018/2/23 | Add Error Code | SunmiPaySDKService1.0.4/SunmiPayHardwareService3.0.44 |  |
| 3.0.3 | 2018/3/21 | Add setSysParam(), Add TransPreProcess() | SunmiPaySDKService1.0.6/SunmiPayHardwareService3.1.2 |  |
| 3.0.4 | 2018/4/12 | Add smartCardExchange()parameter definition. | SunmiPaySDKService1.0.6/SunmiPayHardwareService3.1.2 |  |
| 3.0.5 | 2018/5/3 | Support SM4 algorithm. | SunmiPayHardwareService_v3.2.07 |  |
| 3.0.6 | 2018/5/18 | Add the SysParam constant definition of the keyword "RESERVED" on and off the buzzer instructions. | SunmiPayHardwareService_v3.2.07 |  |
| 3.0.7 | 2018/6/5 | Add mifareReadBlock() blockData format definition | SunmiPayHardwareService_v3.2.07 |  |
| 3.0.8 | 2018/8/16 | Support DUKPT. | SunmiPayHardwareService_v3.2.18 |  |
| 3.0.9 | 2018/8/31 | Add detectCard() and read card info | SunmiPayHardwareService_v3.2.19 |  |
| 3.1.0 | 2018/9/21 | Modify the description for apduCommand() | SunmiPayHardwareService_v3.2.19 |  |
| 3.2.0 | 2018/10/12 | Support V2 api. | SunmiPayHardwareService_v3.2.20 |  |
| 3.2.1 | 2018/10/22 | Add error code | SunmiPayHardwareService_v3.2.20 |  |
| 3.2.2 | 2018/10/24 | Add API: smartCardExchange() and smartCardExchangeNISO() | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.3 | 2018/11/01 | 1.Fix issue: wrong params order of dataEncrypt () 2.Modify method comments in SunmiPayKernel | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.4 | 2018/11/02 | Modify Class EMVTransDataV2 | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.5 | 2018/11/05 | 1.add API: initEmvProcess() 2.Modify get/Set TLV data API 3.Modify EMV flow chart | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.6 | 2018/11/07 | 1.Delete fields of EMVTransDataV2 2.Update EMV flow chart | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.7 | 2018/11/15 | 1.Update describe of PinPadConfigV2 2.Modify the getKeyCheckValue description 3.EMV procedure add interfaces: onAppFinalSelect(),importAppFinalSelectStatus(). 4.remove interface apduCommandNISO() | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.8 | 2018/11/16 | 1.change name smartCardExChangeNISO()to transmitApdu() 2.transmitApdu() return card data directly(remove fields len,swa,swb) | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.9 | 2018/11/19 | Modify interface transmitApduset, change max size of sendBuff, recvBuff to 255B | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.10 | 2018/11/22 | Change type of certType to int in interface EMVListenerV2. onCertVerify() | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.11 | 2018/11/24 | 1.add interface EMVListenerV2. onRequestSignature(),importSignatureStatus() 2.add error code -50027(Signature error) 3.Modify interface buzzerOnDevice() | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.12 | 2018/11/29 | 1.add interface EMVListenerV2.onCardDataExchangeComplete() | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.13 | 2018/12/04 | 1.add interface EMVListenerV2. onConfirmationCodeVerified() 2.add error code -50028 3.update emv flow chart | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.14 | 2018/12/08 | 1.update interface EMVListenerV2.onWaitAppSelect(),change paramter type to List<EMVCandidateV2> 2.modify interface EMVOptV2.importOnlineProcStatus() | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.15 | 2018/12/12 | 1.Add code depiction of callback EMVListenerV2.onTransResult() 2.Modify depiction of parameter status in interface importOnlineProcStatus() | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.16 | 2018/12/13 | Increase -20001—-20006 error code | SunmiPayHardwareService_v3.3.0 |  |
| 3.2.17 | 2019/01/07 | Add RSA interfaces | SunmiPayHardwareService_v3.3.16 |  |
| 3.2.18 | 2019/01/17 | 1.Add interface removeRSAKey() 2.Remove the T=0,T=1 description i-n interface transmitApdu() | SunmiPayHardwareService_v3.3.17 |  |
| 3.2.19 | 2019/01/21 | Update the keyIndex params illustr-ation of RSA/Cetificate related interfaces | SunmiPayHardwareService_v3.3.17 |  |
| 3.2.20 | 2019/03/18 | 1.Update interface saveKeyDukpt(), add code to generate initialize KSN 2.Add interface dukptGetInitKsn() 3.Update interface transmitApdu(),support contact card. 4.Update interface apduCommand(), modify fields Lc,Le,outLen value range of param ApduSendV2, ApduRecvV2 | SunmiPayHardwareService_v3.3.28 |  |
| 3.2.21 | 2019/03/19 | 1.Update interface saveKeyDukpt(),remove generate initialize ksn code 2.Update interface dukptGetInitKSN() 3.Add interface EMVOptV2.abortTransactProcess() | SunmiPayHardwareService_v3.3.29 |  |
| 3.2.22 | 2019/04/10 | 1.add MifarePlus SL3 card data exchange interfaces 2.add SLE4442/4428 card data exchange interfaces 3.add AT24C01/02/04/08/16/32/64/128/256/512 card data exchange Interfaces 4.add embedded doc [Sunmi PinBlock format] | SunmiPayHardwareService_v3.3.32 |  |
| 3.2.23 | 2019/05/29 | 1.add interface EMVOptV2.transactProcessEx(),EMVOptV2.importDataExchangeStatus() 2.add interface EMVListenerV2.onRequestDataExchange() | SunmiPayHardwareService_v3.3.40 |  |
| 3.2.24 | 2019/07/06 | 1.add at88scxx card data exchange interfaces 2.add interface transmitApduEx(),support customized Miafre card APDU data exchange 3.remove firmware version of revision history column [The appropriate SDK version] 4.Add interface sysGetRandom() 5.Add RSA signing/verify signature interaces | SunmiPayHardwareService_v3.3.46 |  |
| 3.2.25 | 2019/07/30 | 1.Add key inject interfaces injectPlaintextKey(),injectCiphertextKey() 2.CheckCardCallbackV2 add callback methods findICCardEx(),findRFCardEx(),onErrorEx() 3.Modify CheckCardCallbackV2. findMagCard(), add key cardType, trackErrorCode,etc in param info. 4.Add interface EMVOptV2.queryECBalance() | SunmiPayHardwareService_v3.3.48 |  |
| 3.3.26 | 2019/09/16 | 1.Modify illustration of interface checkCard() 2.AidV2 Add fields clsStatusCheck, zeroCheck 3.EMVOptV2 add interfaces addDrlLimitSet(), deleteDrlLimitSet(), setTermParamEx() to support DRL | SunmiPayHardwareService_v3.3.52 |  |
| 3.2.27 | 2019/10/15 | 1.Add system parameter constants: EMVVersion,PaypassVersion,PaywaveVersion,QPBOCVersion,EntryVersion,MirVersion,JCBVersion,PAGOVersion,EmvKernelCheckSum 2.Change the hit text of input PIN 3.Add Error code -70001~ -70006 | SunmiPayHardwareService_v3.3.53 |  |
| 3.2.28 | 2019/10/28 | 1.EMVOptV2 add interface queryAidCapkLis()、transactPreProcess() 2.PinPadOptv2 add interface cancelInputPin() 3. add cardCategory, atqa in radio card check card call back. | SunmiPayHardwareService_v3.3.55 |  |
| 3.2.29 | 2019/11/29 | 1. Security module add interfaces dataEncryptDukptEx(),dataDecryptDukptEx(),calcMacDukptEx(),verifyMacDukptEx(),saveTR31Key(),saveCiphertextKeyRSA(),saveRSAKey() 2.Add system Param key：AEVersion 3.Add new EMV flowType, value is 0x04 4.EMV module add interfaces addRevocList()、deleteRevocList()、 sysSetTime()、sysGetTime()、clearData() 5.Modify param name in interfaces signingRSA(),verifySignatureRSA() | SunmiPayHardwareService_v3.3.58 |  |

- 2. Abstract

- 2.1 Introduction

SunmiPaySDK is a set of interface which is based on firmware encapsulation and is close to Java developer to call hardware. Through this SDK, developer can quickly call the corresponding firmware interface of Sunmi financial instruments and realize the development of its own business logic. SDK mainly includes: terminal information basic module, card operation module, password keyboard module, EMV module, security module.

This document is the SunmiPaySDK V2 interface documentation. Compared to the V1 interface, the V2 interface makes it easier to develop this understanding and call.

- 2.2 SDK Integration

2.2.1 Quick integration

This document is an integrated development guide for the SUNMI’s POS. To guide the use method of SDK, It requires the reader to have already familiar with the basic use method of IDE, have certain Android programming basis, familiar with financial related specifications, processes and related concepts (key, pinblock, pan,emv, MAC, etc.). At present, SDK only supports the development of Sunmi P1N, P1-4G, P2 Pro, P2 Lite. Please read this document carefully before using SDK. Check the following prerequisites before use.

- Model number is P1N or P1-4G or P2 Pro or P2 Lite.(Setting-About device-Model number)

- Go to Setting—>APP—>top right cornor—>show system

SunmiPayHardwareService_v3.3.0_release.apk or the latest version.

- Android studio quick integrate，Put the PayLib-release-xxx.aar package under the libs folder.

repositories { flatDir { dirs 'libs' } } dependencies { ...... compile(name: 'PayLib-release-xxx', ext: 'aar') }

Build the Project again when finished.

2.2.2 Android version and IDE version supported by the SDK

SDK only support API-19(Android 4.4) or the latest version.

SDK only support Android studio、Intellij.

- 3. API

- 3.1 SunmiPayKernel SDK Operation Object

3.1.1 Get an instance of SunmiPayKernel

| Prototype | SunmiPayKernel getInstance() |  |
| --- | --- | --- |
| Feature | Get kernel instance object of SDK |  |
| Parameter | [in] | None |
| Return | SunmiPayKernel |  |
| Comment | None |  |

3.1.2 Connect to PaySDK

| Prototype | initPaySDK (Context context,ConnectCallback connectCallback) |  |
| --- | --- | --- |
| Feature | Initialize Pay SDK. |  |
| Parameter | context[in] | Context |
|  | connectCallback[in] | Callback of connection status. Refer to ConnectCallback |
| Return | None |  |
| Comment | It is recommended to call this method in your app’s Application class, if failure to call this method, any invoking of SDK API method may throw NullPointerException. |  |

3.1.3 Disconnect to PaySDK

| Prototype | destroyPaySDK () |  |
| --- | --- | --- |
| Feature | Destroy the Pay SDK. |  |
| Parameter | [in] | None |
| Return | None |  |
| Comment | This method will disconnect your app to Pay SDK. After calling this method, any function module in SunmiPayKernel will be set to null, and any invoke of SDK API method will throw NullPointerException. To prevent a memory leak, call this methos in Activity’s onDestroy() is best practice. |  |

3.1.4 ConnectCallback

3.1.4.1 Connect to PaySDK callback

| Prototype | onConnectPaySDK () |  |
| --- | --- | --- |
| Feature | Callback method, indicate connect to PaySDK success |  |
| Parameter | [in] | None |
| Return | None |  |
| Comment | Any SDK API method calls must be made after receiving this callback. After initializing the SDK, developers should pay attention to this method. When this method called, it indicates the SDK was initialized successfully, and the SDK related API can be invoked rightly. |  |

3.1.4.2 Disconnect to PaySDK callback

| Prototype | onDisconnectPaySDK () |  |
| --- | --- | --- |
| Feature | Disconnect the PaySDK |  |
| Parameter | [in] | None |
| Return | None |  |
| Comment | When this method called, it indicates your app has disconnected to Pay SDK, any invoke of SDK API method may produce Exception. To use SDK API, please call method InitPaySDK() firstly. |  |

- 3.2 Public member variable

After connected to PaySDK, in SunmiPayKernel, there are some function modules can be used to interactive with PaySDK, as following table showing:

| Variable name | Description | Comment |
| --- | --- | --- |
| mPinPadOptV2 | Pinpad operation module | Includes PinPad API |
| mBasicOptV2 | Basic operation module | Get basic info，control LED，control Buzzer. |
| mReadCardOptV2 | Read card module | Read card |
| mEMVOptV2 | EMV module | Emv operation API |
| mSecurityOptV2 | Security module | Save key，mac calculation，encrypt data |

- 3.3 Basic operation module

3.3.1 Get system parameter

| Prototype | String getSysParam(String key) |  |
| --- | --- | --- |
| Feature | Read the properties of the system resource key through the user parameter keyword. |  |
| Parameter | key[in] | Refer to AidlConstantsV2. SysParam |
| Return | Attribute values queried |  |
| Comment | If the required attribute value does not exist, then return "NULL". |  |

3.3.2 Set system parameter

| Prototype | int setSysParam(String key, String value) |  |
| --- | --- | --- |
| Feature | Set the properties of the system resource key through the user parameter keyword. |  |
| Parameter | key[in] | Refer to AidlConstantsV2. SysParam（only support keyword: RESERVED） |
|  | value[in] | Keyword’s value |
| Return | 0: Success Other value: Fail |  |
| Comment | Only support keyword: RESERVED |  |

3.3.3 Control Buzzer

| Prototype | void buzzerOnDevice(int count, int freq, int duration, int interval) |  |
| --- | --- | --- |
| Feature | Control the Buzzer on the device. |  |
| Parameter | count[in] | Beep times (0~100) |
|  | freq[in] | The frequency of tone (unit: Hz) |
|  | duration[in] | The duration time (unit: ms) |
|  | interval[in] | The time interval between two beeps (unit: ms) 0~10000 |
| Return | None |  |
| Comment | None |  |

3.3.4 Control Led

| Prototype | int ledStatusOnDevice (int ledIndex, int ledStatus) |  |
| --- | --- | --- |
| Feature | Control Led status on device |  |
| Parameter | ledIndex[in] | Refer to LedLight definition: AidlConstantsV2.LedLight |
|  | ledStatus[in] | LED status, 1-LED off; 0-LED on. |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.3.5 Set screen mode

| Prototype | int setScreenMode(int mode) |  |
| --- | --- | --- |
| Feature | Set the screen exclusive, disable the bottom navigation bar and the SystemUI drop-down box, and disable the volume key. |  |
| Parameter | mode[in] | Set screen exclusive mode.1：Set screen exclusive，-1: Set screen exclusive exit. |
| Return | 0: Success Other value: Fail |  |
| Comment | Setting up screen exclusivity usually needs an appropriate time to cancel. It is recommended that the user use this interface with the power management lock to keep the screen always bright and unlocked. Otherwise, the screen exclusivity can be maintained after the information screen appears. Only by unplugging the battery and restarting the device can the screen exclusivity be removed. |  |

3.3.6 Get random data

| Prototype | int sysGetRandom(byte[] randData, int len) |  |
| --- | --- | --- |
| Feature | Get a specific length random data from SDK |  |
| Parameter | randData[out] | Buffer, store the random data |
|  | len[in] | The data length, range 0~256 |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

- 3.4 Card operation module

3.4.1 ReadCardOpt

3.4.1.1 Check card

| Prototype | void checkCard(int cardType, CheckCardCallbackV2 callback, int timeout) |  |
| --- | --- | --- |
| Feature | For card checking, magnetic stripe card, IC card and NFC card are supported. After checking, the card type will be put into CheckCardCallbackV2. |  |
| Parameter | cardType[in] | Composite card types, support checking multi cards. This param can be a composite value of CardType.getValue(), for example, check MAGNETIC,NFC,IC at the same time, set this param as: CardType.MAGNETIC.getValue()¦CardType.NFC.getValue()¦ CardType.IC.getValue() |
|  | callback[in] | Check card callback. Refer to CheckCardCallbackV2 |
|  | timeout[in] | Timeout (unit: second). Effective time range: 1-120s |
| Return | None |  |
| Comment | No distinction between bank card and non bank card. |  |

3.4.1.2 Cancel check card

Interface instructions: Artificial return must call cancelCheckCard(), terminate the underlying blocking thread, otherwise the next execution function will fail (for example, click the physical return key, click the interface navigation bar return key to call the function)

| Prototype | void cancelCheckCard(); |  |
| --- | --- | --- |
| Feature | Cancel check card |  |
| Parameter | [in] | None |
| Return | None |  |
| Comment | The function needs to be called when the CheckCard is not returned (CheckCardCallbackV2 Success or Failed Interface is not callback) and leaves the interface. |  |

3.4.1.3 APDU command exchange

| Prototype | int apduCommand (int cardType, ApduSendV2 send, ApduRecvV2 recv) |  |
| --- | --- | --- |
| Feature | IC card operation function. This function support the general interface protocol for IC card.(T=0 and T=1) This support the general interface protocol for contactless card. (T=CL) |  |
| Parameter | cardTypy[in] | Card type currently in operation |
|  | send[in] | class ApduSendV2 { byte[] command; // Command[] = {CLA，INS，P1，P2}. short lc; // Length of dataIn (0~256) byte[] dataIn; // Data of the IC card which you want to send, max length is 256 bytes. short le; // Expected length of the returned data (0~256) } |
|  | recv[out] | class ApduRecvV2 { short outLen; //Actual data length returned from IC card (0~256) byte[] outData; // Data returned from IC card. max length is 256 bytes. byte swa; // Status byte 1 byte swb; // Status byte 2 } |
| Return | 0: Success Other value: Fail |  |
| Comment | For variable format of ApduSendV2 , refer to the doc apdu format and implement in sunmi way |  |

3.4.1.4 APDU command exchange (not recommend)

| Prototype | int smartCardExchange (int cardType, byte[] apduend, byte[] apduRecv) |  |
| --- | --- | --- |
| Feature | IC card operation function. This function support the general interface protocol for IC card.(T=0 and T=1) This support the general interface protocol for contactless card. (T=CL) |  |
| Parameter | cardTypy[in] | Card type currently in operation |
|  | apduSend[in] | The APDU command which to send, format is: Command(4B)+LC(1B, value is len)+inData(len B)+LE(1B) LC is unsigned, range is 0~255 |
|  | apduRecv[out] | The out data buffer, apduRecv.length>=260，receive data format is： outLen(2B, Big endian,value is len)+outData(len B)+SWA(1B)+SWB(1B) |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.4.1.5 Transmit APDU command to card

| Prototype | int transmitApdu (int cardType, byte[] sendBuff, byte[] recvBuff) |  |
| --- | --- | --- |
| Feature | Transmit the sendBuff to card directly. This function support the general interface protocol for IC card.(T=0 and T=1), and also support the general interface protocol for contactless card. (T=CL) |  |
| Parameter | cardType[in] | Card type currently in operation |
|  | sendBuff[in] | Data which want to transmit to card. Max length is 255B |
|  | recvBuff [out] | Data received from card. The max receive data length is 255B, recvBuff.lenght>=255 |
| Return | >=0: The valid data length in recvBuff <0: Error code |  |
| Comment | None |  |

3.4.1.6 Close contact and contactless module

| Prototype | int cardOff(int cardType) |  |
| --- | --- | --- |
| Feature | Power off contact IC card or turn off carrier of contactless card. |  |
| Parameter | cardType[in] | Card type, support multiple card for checking card. Refer to: AidlConstantsV2.CardType.getValue().Fill in the card type according to the card type in checkCard, one at a time. |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.4.1.7 Check if card exist on slot

| Prototype | int getCardExistStatus(int cardType) |  |
| --- | --- | --- |
| Feature | Check if card exist on slot |  |
| Parameter | cardType[in] | Card type (non-composite type), only supports: NFC、IC、PSAM0、PSAM1 Can only be one of 4 types at a time. |
| Return | AidlConstantsV2.CardExistStatus CARD_ABSENT = 0x01; // Card don't exist. CARD_PRESENT = 0x02; // Card exist. |  |
| Comment |  |  |

3.4.1.8 Mifare Classic

3.4.1.8.1 Verify password of M1 card sector

| Prototype | int mifareAuth(int keyType, int block, byte[] key) |  |
| --- | --- | --- |
| Feature | Submit password A or B of responding read-write block when verifying the access of M1 card. |  |
| Parameter | keyType[in] | Specified password type: 0 --- submit password A 1 --- submit password B |
|  | block[in] | Block number for specified access. The available range is 0~63 for M1 card with 1K. |
|  | key[in] | Point to submitted password buffer(6 Bytes) |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.4.1.8.2 M1 card read block data

| Prototype | int mifareReadBlock( int block, byte[] outData) |  |
| --- | --- | --- |
| Feature | Read the contents in the specified block of M1 card. (16 bytes) |  |
| Parameter | block[in] | Block number for specified access. The available range is 0~63 for M1 card with 1K. |
|  | outData [out] | Block data. |
| Return | >=0 Length of outData <0 Error code |  |
| Comment | None |  |

3.4.1.8.3 M1 card write block data

| Prototype | int mifareWriteBlock( int block, byte[] data) |  |
| --- | --- | --- |
| Feature | Write specified content in specified block for M1 card (16 bytes). |  |
| Parameter | block[in] | Block number for specified access. The available range is 0~63 for M1 card with 1K. |
|  | data[in] | Block data. |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.4.1.8.4 M1 card operate sector data (Increment operation)

| Prototype | int mifareIncValue( int block, byte[] value) |  |
| --- | --- | --- |
| Feature | Charge for wallet of M1 card. |  |
| Parameter | block[in] | Use to assign the number of block where the wallet is in it. The available range is 0~63 for M1 card with 1K. |
|  | value[in] | Point to first address of buffer for the amount of charging. Totally 4 bytes for the amount and the lower byte is at the beginning. |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |
| Example |  |  |

3.4.1.8.5 M1 card operate sector data (decrement operation)

| Prototype | int mifareDecValue( int block, byte[] value) |  |
| --- | --- | --- |
| Feature | Charge back for wallet of M1 card. |  |
| Parameter | block[in] | Use to assign the number of block where the wallet is in it. The available range is 0~63 for M1 card with 1K. |
|  | value[in] | Point to first address of buffer for the amount of charging back. Totally 4 bytes for the amount and the lower byte is at the beginning. |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |
| Example |  |  |

3.4.1.9 Mifare Ultralight C

3.4.1.9.1 Verify Password of Mifare Ultralight C Card Sector

| Prototype | int mifareUltralightCAuth(byte[] authKey) |  |
| --- | --- | --- |
| Feature | Submit password A or B of responding read-write block when verifying the access of Mifare ultralight c. |  |
| Parameter | authKey[in] | Point to submitted password buffer |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.4.1.9.2 Mifare Ultralight C Read Block Data

| Prototype | int mifareUltralightCReadData(int block,byte[] outData) |  |
| --- | --- | --- |
| Feature | Read the contents in the specified block of Mifare ultralight c |  |
| Parameter | block[in] | Block number |
|  | outData[out] | Block data. |
| Return | >=0 Length of outData <0 Error code |  |
| Comment | None |  |

3.4.1.9.3 Mifare Ultralight C Write Block Data

| Prototype | int mifareUltralightCWriteData(int block,byte[] data) |  |
| --- | --- | --- |
| Feature | Write specified content in specified block for Mifare ultralight c. |  |
| Parameter | block[in] | Block number |
|  | data[in] | Block data. |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.4.1.10 Mifare Plus SL3

3.4.1.10.1 Mifare Plus read block data

| Prototype | int mifarePlusReadBlock(int block, byte[] key, byte[] outData) |  |
| --- | --- | --- |
| Feature | Mifare plus read block data. |  |
| Parameter | block[in] | Block number for specified access. The available range is depends on card, eg: MF1PLUS80x contains 32 4block sectoers and 8 16block secoters, each block size is 16 bytes, total memory is 32*4*16+8*16*16=4096, total block block count is 32*4+8*16=256, block range is 00~FF. |
|  | key[in] | The block key, 16 bytes. |
|  | outData[out] | Buffer, store the block data, outData.lenght>=16 |
| Return | >=0: The valid data length of outData <0: Error code |  |
| Comment | None |  |
| Example | None |  |

3.4.1.10.2 Mifare Plus write block data

| Prototype | int mifarePlusWriteBlock(int block, byte[] key, byte[] data) |  |
| --- | --- | --- |
| Feature | Mifare plus write block data. |  |
| Parameter | block[in] | Block number for specified access. The available range is depends on card, eg: MF1PLUS80x contains 32 4block sectoers and 8 16block secoters, each block size is 16 bytes, total memory is 32*4*16+8*16*16=4096, total block block count is 32*4+8*16=256, block range is 00~FF. |
|  | key[in] | The block key, 16 bytes. |
|  | data[in] | The data to be write, 16 bytes |
| Return | 0: Success Other value: Error code |  |
| Comment | None |  |
| Example | None |  |

3.4.1.10.3 Mifare Plus change block key

| Prototype | int mifarePlusChangeBlockKey(int block, byte[] oldKey, byte[] newKey) |  |
| --- | --- | --- |
| Feature | Mifare plus change block key |  |
| Parameter | block[in] | Block number for specified access. The available range is depends on card, eg: MF1PLUS80x contains 32 4block sectoers and 8 16block secoters, each block size is 16 bytes, total memory is 32*4*16+8*16*16=4096, total block block count is 32*4+8*16=256, block range is 00~FF. |
|  | oldKey [in] | The old key, 16 bytes. |
|  | newKey [in] | The new key, 16 bytes |
| Return | 0: Success Other value: Error code |  |
| Comment | None |  |
| Example | None |  |

3.4.1.11 SLE4442/SLE4428

3.4.1.11.1 SLE verify password

| Prototype | int sleAuthKey(byte[] key) |  |
| --- | --- | --- |
| Feature | SLE verify password |  |
| Parameter | key[in] | The password,SLE4442 is 3 bytes,SLE4428 is 2 bytes,default value is all ‘F’ |
| Return | 0: Success Other value: Error code |  |
| Comment | None |  |
| Example | None |  |

3.4.1.11.2 SLE change password

| Prototype | int sleChangeKey(in byte[] newKey) |  |
| --- | --- | --- |
| Feature | SLE change password |  |
| Parameter | newKey[in] | The new password,SLE4442 is 3 bytes, SLE4428 is 2 bytes |
| Return | 0: Success Other value: Error code |  |
| Comment | This method only useful after calling sleAuthKey() sucess. |  |
| Example | None |  |

3.4.1.11.3 SLE read data

| Prototype | int sleReadData(int startAddress, int length, byte[] outData) |  |
| --- | --- | --- |
| Feature | SLE read a successive memory segment data |  |
| Parameter | startAddress [in] | The start address |
|  | length[in] | The data length of read |
|  | outData[out] | Buffer, store the read data |
| Return | >=0: The valid data length of outData <0: Error code |  |
| Comment | None |  |
| Example | None |  |

3.4.1.11.4 SLE write data

| Prototype | int sleWriteData(int startAddress, byte[] data) |  |
| --- | --- | --- |
| Feature | SLE write a successive memory segment data |  |
| Parameter | startAddress [in] | The start address |
|  | data[in] | The data to write, max length is 253 bytes |
| Return | 0: Success Other value: Error code |  |
| Comment | If card has password, this method only useful after calling sleAuthKey() sucess. |  |
| Example | None |  |

3.4.1.11.5 SLE get remain authentication count

| Prototype | int sleGetRemainAuthCount() |  |
| --- | --- | --- |
| Feature | SLE get remain authentication count |  |
| Parameter | [in] | None |
| Return | >=0: The remain authentication count <0: Error code |  |
| Comment | If the remain authentication count is 0, card are locked and can not verify password or write data. |  |
| Example | None |  |

3.4.1.11.6 SLE write protection memory

| Prototype | int sleWriteProtectionMemory(int startAddress, int length) |  |
| --- | --- | --- |
| Feature | SLE lock a successive memory segment, make it can not be written, no matter verify password success or not |  |
| Parameter | startAddress[in] | The start address |
|  | length[in] | The length of memory |
| Return | 0: Success Other value: Error code |  |
| Comment | None |  |
| Example | None |  |

3.4.1.11.7 SLE read memory protection status

| Prototype | int sleReadMemoryProtectionStatus(int startAddress, int length, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | SLE read a successive memory protection status. |  |
| Parameter | startAddress[in] | The start address |
|  | length[in] | The length of memory |
|  | dataOut[out] | Buffer, store each memory unit locking status, 0-locked, 1-not locked |
| Return | >=0: The valid data length of outData <0: Error code |  |
| Comment | None |  |
| Example | None |  |

3.4.1.12 AT24C01/02/04/08/16/32/64/128/256/512

3.4.1.12.1 AT24C read data

| Prototype | int at24cReadData(int startAddress, int length, byte[] outData) |  |
| --- | --- | --- |
| Feature | AT24C read a successive memory segment data |  |
| Parameter | startAddress[in] | The start address |
|  | length[in] | The length of memory |
|  | dataOut[out] | Buffer, store the read data |
| Return | >=0: The valid data length of outData <0: Error code |  |
| Comment | None |  |
| Example | None |  |

3.4.1.12.2 AT24C write data

| Prototype | int at24cWriteData(int startAddress, byte[] data) |  |
| --- | --- | --- |
| Feature | AT24C write a successive memory segment data |  |
| Parameter | startAddress[in] | The start address |
|  | data [in] | The data to write |
| Return | 0: Success Other value: Error code |  |
| Comment | None |  |
| Example | None |  |

3.4.1.13 AT88SC

3.4.1.13.1 AT88SC verify password

| Prototype | int at88scAuthKey(byte[] key, int rwFlag, int zoneNo) |  |
| --- | --- | --- |
| Feature | AT88SC verify password |  |
| Parameter | key[in] | The password,AT88SC1608 is 3 bytes ,default value is all ‘F’ |
|  | rwFlag[in] | Write/Read flag, 0- The write key,1-The read key |
|  | zoneNo[in] | User zone number, range 0~7 |
| Return | 0: Success Other value: Error code |  |
| Comment | For AT88SC1608, there are 8 user zones(0~7), each zone contains 256 bytes, total 2048 bytes, address range 0~2047(0~7FF) |  |
| Example | None |  |

3.4.1.13.2 AT24C88 change password

| Prototype | int at88scChangeKey(byte[] newKey, int rwFlag, int zoneNo) |  |
| --- | --- | --- |
| Feature | AT88SC change password |  |
| Parameter | newKey[in] | The new password,AT88SC1608 is 3 bytes |
|  | rwFlag[in] | Write/Read flag, 0-The write key, 1-The read key |
|  | zoneNo[in] | User zone number, range 0~7 |
| Return | 0: Success Other value: Error code |  |
| Comment | This method only useful after calling at88scAuthKey() sucess. |  |
| Example | None |  |

3.4.1.13.3 AT88SC read data

| Prototype | int at88scReadData(int startAddress, int length, int zoneFlag, byte[] outData) |  |
| --- | --- | --- |
| Feature | AT88SC read a successive memory segment data |  |
| Parameter | startAddress [in] | The start address |
|  | length[in] | The data length of read |
|  | zoneFlag[in] | The zone flag, 0-Config zone,1-User zone |
|  | outData[out] | Buffer, store the read data |
| Return | >=0: The valid data length of outData <0: Error code |  |
| Comment | None |  |
| Example | None |  |

3.4.1.13.4 AT88SC write data

| Prototype | int at88scWriteData(int startAddress, int zoneFlag, byte[] dataIn) |  |
| --- | --- | --- |
| Feature | SLE write a successive memory segment data |  |
| Parameter | startAddress [in] | The start address |
|  | zoneFlag[in] | The zone flag, 0-Config zone,1-User zone |
|  | data[in] | The data to write, max length is 253 bytes |
| Return | 0: Success Other value: Error code |  |
| Comment | If card has password, this method only useful after calling at88scAuthKey() sucess. |  |
| Example | None |  |

3.4.1.13.5 AT88SC get remain authentication count

| Prototype | int at88scGetRemainAuthCount(int rwFlag, int zoneNo) |  |
| --- | --- | --- |
| Feature | AT88SC get remain authentication count |  |
| Parameter | rwFlag[in] | Write/Read flag, 0-The write key, 1-The read key |
|  | zoneNo[in] | User zone number, range 0~7 |
| Return | >=0: The remain authentication count <0: Error code |  |
| Comment | If the remain authentication count is 0, card are locked and can not verify password or write data. |  |
| Example | None |  |

3.4.1.14 Transmit APDU command to card(extended method)

| Prototype | int transmitApduEx(int cardType, byte[] sendBuff, out byte[] recvBuff) |  |
| --- | --- | --- |
| Feature | Transmit the sendBuff to card directly. This function support the general interface protocol for IC card.(T=0 and T=1), and also support the general interface protocol for contactless card. (T=CL) |  |
| Parameter | cardType[in] | Card type currently in operation |
|  | sendBuff[in] | Data which want to transmit to card. Max length is 255B |
|  | recvBuff [out] | Data received from card. The max receive data length is 255B, recvBuff.lenght>=255 |
| Return | >=0: The valid data length in recvBuff <0: Error code |  |
| Comment | This method is different from transmitApdu only when cardtype is Mifare,the following text show the differences: 1. When card type is Mifare, the first byte(B1) of send data represents the setting of communication parameters: Bit0: 1-enable Rx CRC, 0-disable Rx CRC Bit1: 1-enable Tx CRC, 0-disable Tx CRC Bit2: 0-enable Rx parity, 1-disable Rx parity Bit3: 0-enable Tx parity, 1-disable Tx parity Bit4-bit7: TxLastBits, TxLastBits = 0 - Send whole data of the last byte, TxLastBits = n (n≠0) - Send n bits of the last byte. 2.For interface transmitApdu, param sendBuff should not contains B1. SDK defaults to send 0x03 + sendBuff to card. 3. For interface transmitApduEx, param sendBuff should contains B1, sendBuff [0] is B1,the format of B1 should comply to paragraph 1. |  |

3.4.2 CheckCardCallbackV2

3.4.2.1 Find magnetic card

| Prototype | void findMagCard(Bundle info) |  |
| --- | --- | --- |
| Feature | Find magnetic stripe card. |  |
| Parameter | info[in] | Contains the following data: cardType: int, the card type TRACK1: String, track1 data TRACK2: String, track2 data TRACK3: String, track3 data track1ErrorCode: int, track1 error code track2ErrorCode: int, track2 error code track3ErrorCode: int, track3 error code The track error code can be one of the following values: -1: Track no data -2: Track parity check error -3: Track LRC check error |
| Return | None |  |
| Comment | None |  |

3.4.2.2 Find IC card

| Prototype | void findICCard(String atr) |  |
| --- | --- | --- |
| Feature | Find IC card |  |
| Parameter | atr[in] | The card ATR |
| Return | None |  |
| Comment | None |  |

3.4.2.3 Find radio frequency card

| Prototype | void findRFCard(String uuid) |  |
| --- | --- | --- |
| Feature | Find RF Card |  |
| Parameter | uuid[in] | The card UUID |
| Return | None |  |
| Comment | None |  |

3.4.2.4 Check card error

| Prototype | void onError(int code, string message) |  |
| --- | --- | --- |
| Feature | Check card error callback. |  |
| Parameter | code[in] | Error code |
|  | message[in] | Error message |
| Return | None |  |
| Comment | None |  |

3.4.2.5 Find IC card(extended method)

| Prototype | void findICCardEx(Bundle info) |  |
| --- | --- | --- |
| Feature | Find IC card |  |
| Parameter | info[in] | Contains the following data: cardType: int, the checked card type atr: String, the card ATR |
| Return | None |  |
| Comment | Compare with findICCard(), This method provide more detail info in param [info], Both this method and findICCard() will be called in check card process. Client app can implement one of the two to do process. |  |

3.4.2.6 Find radio frequency card(extended method)

| Prototype | void findRFCardEx(Bundle info) |  |
| --- | --- | --- |
| Feature | Find RF Card |  |
| Parameter | info[in] | Contains the following data: cardType: int, the checked card type uuid: String, the card UUID ats: String, the card ATS cardCategory: int, the card category, ‘A’or ‘B’ atqa: byte[], the card ATQA |
| Return | None |  |
| Comment | Compare with findRFCard(), This method provide more detail info in param [info], Both this method and findRFCard() will be called in check card process. Client app can implement one of the two to do process. |  |

3.4.2.7 Check card error(extended method)

| Prototype | void onErrorEx(Bundle info) |  |
| --- | --- | --- |
| Feature | Check card error callback. |  |
| Parameter | info[in] | Contains the following data: cardType: int, the failed card type code: int, the error code message: String, the error message |
| Return | None |  |
| Comment | Compare with onError(), This method provide more detail info in param [info], Both this method and onError() will be called in check card process. Client app can implement one of the two to do process. |  |

- 3.5 PinPad operation module

3.5.1 PinPadOpt

3.5.1.1 Initialize PinPad

| Prototype | void initPinPad(PinPadConfigV2 config, PinPadListenerV2 listener) |  |
| --- | --- | --- |
| Feature | Initialize PinPad: import configuration parameter and register callback listener. |  |
| Parameter | config[in] | PinPad configuration, refer to PinPadConfigV2 |
|  | listerner[in] | Callbacks (if PinPadType in PinPadConfigV2 uses a built-in password keyboard and only callbacks confirm, cancel, error). Refer to PinPadListenerV2 |
|  | [out] | None |
| Return | None |  |
| Comment | For Sunmi PinBlock format, please refer to Sunmi PinBlock format |  |

3.5.1.2 Cancel input PIN

| Prototype | void cancelInputPin() |  |
| --- | --- | --- |
| Feature | Cancel input PIN. |  |
| Parameter | [in] | None |
|  | [out] | None |
| Return | None |  |
| Comment | None |  |

3.5.2 PinPadListenerV2

Note: This interface is the interface passed between AIDLs and must be implemented in accordance with the Aidl interface when the callback is passed.

3.5.2.1 Press number key

| Prototype | void onPinLength(int len) |  |
| --- | --- | --- |
| Feature | Import the PIN length, which is used for display when input PIN. |  |
| Parameter | len[in] | Length of PIN entered |
| Return | None |  |
| Comment |  |  |

3.5.2.2 Press confirm key

| Prototype | void onConfirm(int type, byte[] pinBlock) |  |
| --- | --- | --- |
| Feature | Return PINblock. When type = 0, pinBlock is null to indicate bypass mode (the user did not enter pin to confirm directly), and returning empty is PinBlock data. When type = 1, pinblock is null, which is meaningless in that case. |  |
| Parameter | type[in] | type = 0, online PIN；type = 1, offline PIN. |
|  | pinBlock [out] | Pin block data |
| Return | None |  |
| Comment | None |  |

3.5.2.3 Press cancel key

| Prototype | void onCancel () |  |
| --- | --- | --- |
| Feature | Cancel PIN input. |  |
| Parameter | [in] | None |
| Return | None |  |
| Comment | None |  |

3.5.2.4 Input pin error

| Prototype | void onError (int errorCode) |  |
| --- | --- | --- |
| Feature | Error message |  |
| Parameter | errorCode [in] | Error code, refer to Error Code Definition |
| Return | None |  |
| Comment | None |  |

- 3.6 Security operation module

3.6.1 Save plaintext key

| Prototype | int savePlaintextKey(int keyType, byte[] keyValue, byte[] checkValue, int keyAlgType, int keyIndex) |  |
| --- | --- | --- |
| Feature | Save plaintext key |  |
| Parameter | keyType[in] | Key type. Refer to: Aidl constants Key type constant definition |
|  | keyValue[in] | Key data |
|  | checkValue[in] | Key check value. If the key check value is null, it does not check (when keyAlgType is SM4, the value of checkValue is 16 bytes). |
|  | keyAlgType[in] | For specifying the type of key that is currently saved, refer to appendix: Aidl constant Key algorithm type.. |
|  | keyIndex[in] | Key index (range 0-19) |
| Return | 0: Success Other value: Fail |  |
| Comment | 1.When saving TMK, Client can specify the encryptIndex as a TMK (which already stored) index, this can support multi-layer TMK. 2.When isEncrypt is false, the decryption key parameter is 1 by default. 3.Client should guarantee key index mutex, that is, if stored a MAC key in index 4, then store a track key in index 4 too, the MAC key will be overwritten by the track key. |  |

3.6.2 Save ciphertext key

| Prototype | int saveCiphertextKey(int keyType, byte[] keyValue, byte[] checkValue, int encryptIndex, int keyAlgType, int keyIndex) |  |
| --- | --- | --- |
| Feature | Save ciphertext key |  |
| Parameter | keyType[in] | Key Type: Reference Appendix: Aidl Constant Key Type Definition. KEY_TYPE_KEK Type cannot be saved in ciphertext |
|  | keyValue[in] | Key data |
|  | checkValue[in] | Key check value. If the key check value is null, it does not check (when keyAlgType is SM4, the value of checkValue is 16 bytes). |
|  | encryptIndex[in] | Used to decrypt the key ciphertext index, note that here is the index of TMK. |
|  | keyAlgType[in] | For specifying the type of key that is currently saved, refer to appendix: Aidl constant Key algorithm type. |
|  | keyIndex[in] | Key index (range 0-19) |
| Return | 0: Success Other value: Fail |  |
| Comment | 1.When saving TMK, Client can specify the encryptIndex as a TMK (which already stored) index, this can support multi-layer TMK. 2.When isEncrypt is false, the decryption key parameter is 1 by default. 3.Client should guarantee key index mutex, that is, if stored a MAC key in index 4, then store a track key in index 4 too, the MAC key will be overwritten by the track key. |  |

3.6.3 Calculate MAC

| Prototype | int calcMac (int keyIndex, int macType , byte[] dataIn, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Calculate MAC |  |
| Parameter | keyIndex[in] | Key index (range 0-19) |
|  | macType[in] | MAC encryption algorithm, refer to: Aidl constant.MAC algorithm type |
|  | dataIn[in] | The original data |
|  | dataOut[out] | MAC result |
| Return | >= 0: MAC data length. < 0: Error code. |  |
| Comment | DataIn is the packet data to be computed. Many MAC algorithms have been implemented in the SDK. The results will be returned through dataOut, and the incoming dataOut will be fixed to 8 bytes long byte such as: byte [] dataOut = new byte [8]. |  |

3.6.4 Encrypt data

| Prototype | int dataEncrypt (int keyIndex , byte[] dataIn, int encryptionMode, byte[] iv, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Encrypt data |  |
| Parameter | keyIndex[in] | Key index (range 0-19) |
|  | datain[in] | The original data |
|  | encryptionMode[in] | Encryption mode, refer to appendix: Aidl constant encryption mode constant. |
|  | iv[in] | The initial vector, the encrypted mode is the ECB space, and the 8 byte vector for other encryption modes. |
|  | dataout[out] | Encryption data |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |
| Example | The effective length returned by dataOut is equal to the length of dataIn. The length of dataIn encrypted data must be an integer multiple of 8. E.g.：byte[] dataIn = new byte[16]; DataOut should also transmit 16 bytes. |  |

3.6.5 Decrypt data

| Prototype | int dataDecrypt(int keyIndex , byte[] dataIn, int encryptionMode, byte[] iv, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Decrypt data |  |
| Parameter | keyIndex[in] | Key index (range 0-19) |
|  | dataIn[in] | Ciphertext data |
|  | encryptionMode[in] | Encryption mode, refer to appendix: Aidl constant encryption mode constant. |
|  | iv[in] | The initial vector, the encrypted mode is the ECB space, and the 8 byte vector for other encryption modes. |
|  | dataOut[out] | plaintext |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |
| Example | The effective length returned by dataOut is equal to the length of dataIn. The length of dataIn encrypted data must be an integer multiple of 8. E.g.：byte[] dataIn = new byte[16]; DataOut should also transmit 16 bytes. |  |

3.6.6 Inject dukpt key

| Prototype | int saveKeyDukpt(int keyType, byte[] keyValue, byte[] checkValue, byte[] ksn, int encryptType, int keyIndex) |  |
| --- | --- | --- |
| Feature | Inject dukpt key |  |
|  | keyType[in] | Key type Contain: BDK- base dispersed key: corresponding to Aidl constant class key type constant KEY_TYPE_DUPKT_BDK. IPEK- initial PIN encryption key: corresponding Aidl constant class key type constant KEY_TYPE_DUPKT_IPEK. |
| Parameter | keyValue[in] | Key data |
|  | checkValue[in] | Key check value(When injecting BDK null values) |
|  | ksn[in] | KSN |
|  | encryptType[in] | Key algorithm type, refer to: Aidl constant Key algorithm type |
|  | keyIndex[in] | Key index (range 0-9) |
| Return | 0: Success <0 Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.7 Calculate mac (Based on the DUKPT)

| Prototype | int calcMacDukpt(int keyIndex, int macType , byte[] dataIn, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Dukpt key calculate mac. |  |
| Parameter | keyIndex [in] | Key index (range 0-9) |
|  | macType [in] | Mac algorithm, refer to: Aidl constant.MAC algorithm type |
|  | dataIn [in] | The original data |
|  | dataOut[out] | Mac result. |
| Return | 0: Success <0 Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.8 Encrypt data (Based on the DUKPT)

| Prototype | int dataEncryptDukpt(int keyIndex, byte[] dataIn, int encryptionMode, byte[] iv, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Dukpt key encrypt data |  |
| Parameter | keyIndex [in] | Key index (range 0-9) |
|  | encryptionMode[in] | Encryption mode, refer to appendix: Aidl constant encryption mode constant. |
|  | iv[in] | The initial vector, the encrypted mode is the ECB space, and the 8 byte vector for other encryption modes. |
|  | dataIn [in] | The original data |
|  | dataOut[out] | Encryption data |
| Return | 0: Success <0 Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.9 Decrypt data (Based on the DUKPT)

| Prototype | int dataDecryptDukpt(int keyIndex, byte[] dataIn, int encryptionMode, byte[] iv, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Dukpt Key decrypt data |  |
| Parameter | keyIndex[in] | Key index (range 0-9) |
|  | encryptionMode[in] | Encryption mode, refer to appendix: Aidl constant encryption mode constant. |
|  | iv[in] | The initial vector, the encrypted mode is the ECB space, and the 8 byte vector for other encryption modes. |
|  | dataIn[in] | Encryption data |
|  | dataOut[in] | Decryption results |
| Return | 0: Success <0 Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.10 Dukpt's KSN increased by 1

| Prototype | int dukptIncreaseKSN(int keyIndex) |  |
| --- | --- | --- |
| Feature | Dukpt's KSN increased by 1 |  |
| Parameter | keyIndex[in] | Dukpt key index (range 0-9) |
| Return | 0: Success <0 Fail |  |
| Comment | Key for Dukpt key system |  |
| Example | None |  |

3.6.11 Dukpt gets the current KSN

| Prototype | int dukptCurrentKSN(int keyIndex, byte[] outKSN) |  |
| --- | --- | --- |
| Feature | Dukpt gets the current KSN |  |
| Parameter | keyIndex[in] | Dukpt key index (range 0-9) |
|  | dataOut[out] | Fixed to 10 bytes. |
| Return | 0: Success <0 Fail |  |
| Comment | Key for Dukpt key system |  |
| Example | None |  |

3.6.12 Get key CheckValue

| Prototype | int getKeyCheckValue(int keySystem, int keyIndex, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Get key CheckValue |  |
| Parameter | keySystem[in] | Key system. Reference appendix: Aidl constant class key system constants |
|  | keyIndex[in] | Key index. When keySystem is SEC_DUKPT, the index range is 0-9 and keySystem is SEC_MKSK, and the index range is 0-19. |
|  | dataOut[out] | Fixed to 4 bytes |
| Return | =0：Success <0 失败 |  |
| Comment | The key index obtained by injecting the BDK is the CheckValue of the BDK corresponding to the IPEK. |  |
| Example | None |  |

3.6.13 Get TUSN encrypted data (Only for the Chinese market)

| Prototype | int getTUSNEncryptData (String dataIn, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Get TUSN encrypted data |  |
| Parameter | dataIn[in] | Used to calculate the scatter value of ciphertext |
|  | dataOut[out] | Generated ciphertext |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |
| Example | DataIn encryption random factor (bank card for card number 6, sweep code category for the code after 6). DataOut is fixed into 4 byte byte[] such as: byte[] dataOut = new byte[4]; |  |

3.6.14 Generate RSA key pair

| Prototype | int generateRSAKeys(int pubKeyIndex, int pvtKeyIndex, int keysize, String pubExponent) |  |
| --- | --- | --- |
| Feature | Generate RSA public/private key pair. |  |
| Parameter | pubKeyIndex [in] | Public key stored index, range is [0,19] |
|  | pvtKeyIndex [in] | Private key stored index, range is [0,19] |
|  | keysize [in] | The key size,512~65536,Unit: bit, such as 512,1024,etc |
|  | pubExponent [in] | The public exponent |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.15 Get RSA public key

| Prototype | int getRSAPublicKey(int pubKeyIndex, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Get X509 encoded RSA public key data |  |
| Parameter | pubKeyIndex [in] | The stored index of public key, range is [0,19] |
|  | dataOut[out] | The out buffer, store public key data, ANS.1 X509 standard format encoding |
| Return | >=0: The valid data length of dataOut <0: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.16 Encrypt data with RSA

| Prototype | int dataEncryptRSA(String transformation, int keyIndex, byte[] dataIn, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Encrypt data with RSA |  |
| Parameter | transformation [in] | The transformation used in cipher process. Refer to: RSA transformation definition |
|  | keyIndex [in] | The encrypt key index, range is [0,19] |
|  | dataIn [in] | Data will be encrypted |
|  | dataOut [out] | Out buffer, store encrypted data |
| Return | >=0: The valid data length of dataOut <0: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.17 Decrypt data with RSA

| Prototype | int dataDecryptRSA(String transformation, int keyIndex, byte[] dataIn, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Decrypt data with RSA |  |
| Parameter | transformation [in] | The transformation used in cipher process. Refer to: RSA transformation definition |
|  | keyIndex [in] | The decrypt key index, range is [0,19] |
|  | dataIn [in] | Data will be decrypted |
|  | dataOut[out] | Out buffer, store decrypted data |
| Return | >=0: The valied length of dataOut <0: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.18 Remove RSA key

| Prototype | int removeRSAKey (int keyIndex) |  |
| --- | --- | --- |
| Feature | Remove stored RSA public or private key, range is [0,19] |  |
| Parameter | keyIndex [in] | The stored index of RSA key, range is [0,19] |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.19 Store certificate

| Prototype | int storeCertificate(int certIndex, byte[] certData) |  |
| --- | --- | --- |
| Feature | Store certificate |  |
| Parameter | certIndex [in] | The stored index of certificate, range is [0,19] |
|  | certData [in] | The certificate data |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.20 Get certificate

| Prototype | int getCertificate(int certIndex, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Get stored certificate |  |
| Parameter | certIndex [in] | The stored index of certificate, range is [0,19] |
|  | dataOut [out] | Out buffer, store certificate data |
| Return | >=0: The valied data length of dataOut <0: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.21 Get initialized KSN

| Prototype | int dukptGetInitKSN (byte[] outKSN) |  |
| --- | --- | --- |
| Feature | Get the initialized KSN |  |
| Parameter | outKSN [out] | The initialized KSN |
| Return | >=0: The valied data length of outKSN <0: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.22 RSA algorithm signing data

| Prototype | int signingRSA(String signAlg, int pvtKeyIndex, byte[] dataIn, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | RSA signing data |  |
| Parameter | signAlg [in] | The signing algorithm. Refer to: RSA signature algoritm definition |
|  | pvtKeyIndex [in] | The RSA private key index |
|  | dataIn [in] | The data to be signed |
|  | dataOut [out] | Buffer, store the signature data. |
| Return | >=0: The valied data length of dataOut <0: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.23 RSA algorithm verify signature

| Prototype | int verifySignatureRSA(String signAlg, byte[] pubKey, byte[] srcData, byte[] signature) |  |
| --- | --- | --- |
| Feature | RSA verify sinature |  |
| Parameter | signAlg [in] | The signing algorithm. Refer to: RSA signature algoritm definition |
|  | pubKey [in] | The RSA public key, ANS.1 X509 DER encoded |
|  | srcData [in] | The source data before signed (raw data) |
|  | signature [in] | The signature data. |
| Return | =0: success <0: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.24 Inject plaintext key

| Prototype | int injectPlaintextKey(String targetPkgName, int keyType, byte[] keyValue, byte[] checkValue, int keyAlgType, int keyIndex) |  |
| --- | --- | --- |
| Feature | Save plaintext key |  |
| Parameter | targetPkgName[in] | The target App(App which use the injected key) package name |
|  | keyType[in] | Key type. Refer to: Aidl constants Key type constant definition |
|  | keyValue[in] | Key data |
|  | checkValue[in] | Key check value. If the key check value is null, it does not check (when keyAlgType is KEY_ALG_TYPE_SM4, the value of checkValue is 16 bytes). |
|  | keyAlgType[in] | For specifying the type of key that is currently saved, refer to appendix: Aidl constant Key algorithm type. |
|  | keyIndex[in] | Key index (range 0-19) |
| Return | 0: Success Other value: Fail |  |
| Comment | 1. Client App should make sure keyIndex is unique for different key 2. The inject key App must has the same signature with the target App(App which use the injected key), or the injected key will not be accessible for target App. |  |

3.6.25 Inject ciphertext key

| Prototype | int injectCiphertextKey(String targetPkgName,int keyType,byte[] keyValue,byte[] checkValue, int encryptIndex, int keyAlgType, int keyIndex) |  |
| --- | --- | --- |
| Feature | Save ciphertext key |  |
| Parameter | targetPkgName[in] | The target App(App which use the injected key) package name |
|  | keyType[in] | Key Type: Reference Appendix: Aidl Constant Key Type Definition. KEY_TYPE_KEK Type cannot be saved in ciphertext |
|  | keyValue[in] | Key data |
|  | checkValue[in] | Key check value. If the key check value is null, it does not check (when keyAlgType is KEY_ALG_TYPE_SM4, the value of checkValue is 16 bytes). |
|  | encryptIndex[in] | Used to decrypt the key ciphertext index, note that here is the index of TMK. |
|  | keyAlgType[in] | For specifying the type of key that is currently saved, refer to appendix: Aidl constant Key algorithm type. |
|  | keyIndex[in] | Key index (range 0-19) |
| Return | 0: Success Other value: Fail |  |
| Comment | 1. Client App should make sure keyIndex is unique for different key 2. The inject key App must has the same signature with the target App(App which use the injected key), or the injected key will not be accessible for target App. |  |

3.6.26 Encrypt data (Based on the DUKPT, extended method)

| Prototype | int dataEncryptDukptEx(int keySelect, int keyIndex, byte[] dataIn, int encryptionMode, byte[] iv, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Dukpt key encrypt data |  |
| Parameter | keySelect[in] | The Dukpt key select, refer to Aidl constant dukpt key select |
|  | keyIndex[in] | Key index (range 0-9) |
|  | encryptionMode[in] | Encryption mode, refer to appendix: Aidl constant encryption mode constant. |
|  | iv[in] | The initial vector, the encrypted mode is the ECB space, and the 8 bytes vector for other encryption modes. |
|  | dataIn [in] | The original data |
|  | dataOut[out] | Encryption data |
| Return | 0: Success <0: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.27 Decrypt data (Based on the DUKPT, extended method)

| Prototype | int dataDecryptDukptEx(int keySelect, int keyIndex, byte[] dataIn, int encryptionMode, byte[] iv, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Dukpt Key decrypt data |  |
| Parameter | keySelect[in] | The Dukpt key select, refer to Aidl constant dukpt key select |
|  | keyIndex[in] | Key index (range 0-9) |
|  | encryptionMode[in] | Encryption mode, refer to appendix: Aidl constant encryption mode constant. |
|  | iv[in] | The initial vector, the encrypted mode is the ECB space, and the 8 byte vector for other encryption modes. |
|  | dataIn[in] | Encryption data |
|  | dataOut[in] | Decryption results |
| Return | 0: Success <0: Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.28 Calculate mac (Based on the DUKPT, extended method)

| Prototype | int calcMacDukptEx(int keySelect, int keyIndex, int macType, byte[] dataIn, byte[] dataOut) |  |
| --- | --- | --- |
| Feature | Dukpt key calculate mac. |  |
| Parameter | keySelect[in] | The Dukpt key select, refer to Aidl constant dukpt key select |
|  | keyIndex [in] | Key index (range 0-9) |
|  | macType [in] | Mac algorithm, refer to: Aidl constant.MAC algorithm type |
|  | dataIn [in] | The original data |
|  | dataOut[out] | Mac result. |
| Return | 0: Success <0 Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.29 Verify mac (Based on the DUKPT, extended method)

| Prototype | int verifyMacDukptEx(int keySelect, int keyIndex, int macType, byte[] dataIn, byte[] macData) |  |
| --- | --- | --- |
| Feature | Dukpt key verify mac |  |
| Parameter | keySelect[in] | The Dukpt key select, refer to Aidl constant dukpt key select |
|  | keyIndex [in] | Key index (range 0-9) |
|  | macType [in] | Mac algorithm, refer to: Aidl constant.MAC algorithm type |
|  | dataIn [in] | The original data |
|  | macData [out] | Mac data |
| Return | 0: Success <0 Fail |  |
| Comment | None |  |
| Example | None |  |

3.6.30 Save TR31 key

| Prototype | int saveTR31Key(byte[] keyValue, int kbpkIndex, int keyIndex) |  |
| --- | --- | --- |
| Feature | Save a TR31 key |  |
| Parameter | keyValue[in] | Key data |
|  | kbpkIndex[in] | KBPK index, range is 0~19, inclusive |
|  | keyIndex[in] | Key index, range is 0~19, inclusive |
| Return | 0: Success <0: Fail |  |
| Comment | Before save any TR31 key, KBPK should be saved firstly. |  |
| Example | None |  |

3.6.31 Save ciphertext key with decryption key is RSA private key

| Prototype | int saveCiphertextKeyRSA(int keyType, byte[] keyValue, byte[] checkValue, int keyAlgType, int keyIndex, int encryptIndexRSA, String transformation) |  |
| --- | --- | --- |
| Feature | Save ciphertext key with the decryption key is RSA private key |  |
| Parameter | keyType[in] | Key Type: Reference Appendix: Aidl Constant key type Definition. KEY_TYPE_KEK Type cannot be saved in ciphertext |
|  | keyValue[in] | Key data |
|  | checkValue[in] | Key check value. If the key check value is null, it does not check (when keyAlgType is KEY_ALG_TYPE_SM4, the value of checkValue is 16 bytes). |
|  | keyAlgType[in] | For specifying the type of key that is currently saved, refer to appendix: Aidl constant key algorithm type. |
|  | keyIndex[in] | Key index, range is 0~19, inclusive |
|  | encryptIndexRSA[in] | RSA private key index, range is 0~19, inclusive |
|  | Transformation[in] | RSA transformation, refer to appendix: RSA transformation definition |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.6.32 Save RSA key

| Prototype | int saveRSAKey(int keyType, in byte[] keyValue, int keyIndex) |  |
| --- | --- | --- |
| Feature | Save RSA key |  |
| Parameter | keyType[in] | Key Type, 0-RSA public key, 1-RSA private key |
|  | keyValue[in] | Key data, keyType=0- ANS.1 X509 standard format encoding, keyType=1- ANS.1 PKCS#8 standard format encoding |
|  | keyIndex[in] | Key index, range is 0~19, inclusive |
| Return | 0: Success <0: Fail |  |
| Comment | None |  |
| Example | None |  |

- 3.7 EMV operation module

    - EMV methods

3.7.1.1 Add AID parameter

| Prototype | int addAid(AidV2 aid) |  |
| --- | --- | --- |
| Feature | Add or update one AID |  |
| Parameter | aid [in] | Refer to AidV2 |
| Return | 0: Success Other value: Fail |  |
| Comment |  |  |

3.7.1.2 Delete AID parameter

| Prototype | int deleteAid(String tag9F06Value) |  |
| --- | --- | --- |
| Feature | Delete one AID according to the value of tag 9F06，if you want to delete all AIDs，set tag9F06Value as null. |  |
| Parameter | tag9F06Value [in] | The value of tag 9F06 (hex format). |
| Return | 0: Success Other value: Fail |  |
| Comment |  |  |

3.7.1.3 Add CAPK parameter

| Prototype | int addCapk(CapkV2 capk) |  |
| --- | --- | --- |
| Feature | Add or update one CAPK |  |
| Parameter | capk [in] | Refer to CapkV2 |
| Return | 0: Success Other value: Fail |  |
| Comment |  |  |

3.7.1.4 Delete CAPK parameter

| Prototype | int deleteCapk(String tag9F06Value, String tag9F22Value) |  |
| --- | --- | --- |
| Feature | Delete one CAPK according to the value of tag 9F06 and value of tag 9F22, if you want to delete all CAPKs, set tag9F06Value as null. |  |
| Parameter | tag9F06Value [in] | The value of tag 9F06 (hex format) |
|  | tag9F22Value [in] | The value of tag 9F22 (hex format) |
| Return | 0: Success Other value: Fail |  |
| Comment |  |  |

3.7.1.5 Set terminal parameter

| Prototype | int setTerminalParam(EmvTermParamV2 termParam) |  |
| --- | --- | --- |
| Feature | Set terminal parameter. |  |
| Parameter | termParam [in] | Reference to EMVTermParamV2 |
| Return | 0: Success Other value: Fail |  |
| Comment |  |  |

3.7.1.6 Check whether AID and CAPK exist or not

| Prototype | int isExistCapkAndAid() |  |
| --- | --- | --- |
| Feature | Determine the existence of CAPK and AID. |  |
| Parameter | [in] | None |
| Return | -1: Both AID and CAPK not exist. 0: Both AID and CAPK exist. 1: Only exist AID 2: Only exist CAPK |  |
| Comment | None |  |

3.7.1.7 Initialize EMV process

| Prototype | int initEmvProcess () |  |
| --- | --- | --- |
| Feature | Initialize EMV process |  |
| Parameter | [in] | None |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.7.1.8 Strat transaction process

| Prototype | void transactProcess(EMVTransDataV2 transData, EMVListenerV2 listener) |  |
| --- | --- | --- |
| Feature | Start EMV process |  |
| Parameter | transData [in] | Refer to EMVTransDataV2 |
|  | listener [in] | Refer to EMVListenerV2 |
| Return | None |  |
| Comment |  |  |

3.7.1.9 Read one TLV data from kernel

| Prototype | int getTlv(int opCode,String tag, byte[] outData) |  |
| --- | --- | --- |
| Feature | Read one TLV data from kernel |  |
| Parameter | opCode[in] | Operation code,0-Normal TLV, 1-PayPass TLV, 2-PayWave TLV |
|  | tag [in] | The tag to read (hex format), eg:”95” |
|  | outData[out] | Buffer, store the read data. |
| Return | >=0: The valid data length of outData Other value: Fail |  |
| Comment |  |  |

3.7.1.10 Read TLV data list from kernel

| Prototype | int getTlvList(int opCode,String[] tags, byte[] outData) |  |
| --- | --- | --- |
| Feature | Read TLV data list from kernel |  |
| Parameter | opCode[in] | Operation code,0-Normal TLV, 1-PayPass TLV, 2-PayWave TLV |
|  | tags [in] | The tag list to read (hex format), eg:{“95”,”9F2A”} |
|  | outData [out] | Buffer , store the read data. |
| Return | >=0: The valid data length of outData Other value: Fail |  |
| Comment | If there is no data has been read for a special tag, outData do not contains this tag, eg: tags={“95”,”9F2A”}, if no data has been read for tag“95”, outData may like as “9F2A03A00001”,which don not contains tag “95”. |  |

3.7.1.11 Set TLV data to kernel

| Prototype | void setTlv(int opCode,String tag, String hexValue) |  |
| --- | --- | --- |
| Feature | Set TLV data to kernel |  |
| Parameter | opCode[in] | Operation code,0-Normal TLV, 1-PayPass TLV, 2-PayWave TLV |
|  | tag [in] | The tag to set (hex format) |
|  | hexValue [in] | The value corresponding to this tag (hex format) |
| Return | None |  |
| Comment | None |  |

3.7.1.12 Set TLV data list to kernel

| Prototype | void setTlvList(int opCode,String[] tags, String[] hexValues) |  |
| --- | --- | --- |
| Feature | Set TLV data to kernel |  |
| Parameter | opCode[in] | Operation code,0-Normal TLV, 1-PayPass TLV, 2-PayWave TLV |
|  | tags [in] | The tag list to set (hex format), eg:{“9F1A”,”9F33”} |
|  | hexValues [in] | The values corresponding to this tags list (hex format) eg:{“0156”, “E0F8C8”} |
| Return | None |  |
| Comment | None |  |

3.7.1.13 Import App select result to EMV

| Prototype | void importAppSelect (int selectIndex) |  |
| --- | --- | --- |
| Feature | Import app select result to EMV procedure |  |
| Parameter | selectIndex [in] | The selected app index, start from 0. |
| Return | None |  |
| Comment | None |  |

3.7.1.14 Import app final select result to EMV

| Prototype | void importAppFinalSelectStatus(int status) |  |
| --- | --- | --- |
| Feature | Import app final select result to EMV procedure. |  |
| Parameter | status [in] | Final select result, 0:Success, 1:Fail |
| Return | None |  |
| Comment | None |  |

3.7.1.15 Import card confirm result to EMV

| Prototype | void importCardNoStatus(int status) |  |
| --- | --- | --- |
| Feature | Import card confirm result to EMV procedure. |  |
| Parameter | status [in] | Confirm result, 0:Success, 1:Fail |
| Return | None |  |
| Comment | None |  |

3.7.1.16 Import certificate authorize result to EMV

| Prototype | void importCertStatus(int status) |  |
| --- | --- | --- |
| Feature | Import certificate authorize result to EMV procedure |  |
| Parameter | status [in] | Authorize result, 0: Success, 1: Fail |
| Return | None |  |
| Comment | None |  |

3.7.1.17 Import PIN input result to EMV

| Prototype | void importPinInputStatus(int pinType, int inputResult) |  |
| --- | --- | --- |
| Feature | Import PIN input result to EMV procedure |  |
| Parameter | pinType [in] | PIN type, 0: online PIN, 1: offline PIN |
|  | inputResult [in] | PIN input results. 0: Success, 1: PIN cancels, 2: PIN skipped, 3: PINPAD fails. |
| Return | None |  |
| Comment | None |  |

3.7.1.18 Import online process result to EMV

| Prototype | int importOnlineProcStatus(int status, String[] tags, String[] hexValues, byte[] outData) |  |
| --- | --- | --- |
| Feature | Import online process result to EMV procedure. |  |
| Parameter | status [in] | Online result, 0-Online approval, 1-Online denial, 2- Online failed |
|  | tags[in] | Online data which need to import to kernel,eg: { "71", "72", "91", "8A", "89" } |
|  | hexValues[in] | Values corresponding to each tag in parameter tags |
|  | outData[out] | Result data of kernel process. |
| Return | >=0: The valid data length in outData <0: Error code |  |
| Comment | None |  |

3.7.1.19 Import signature result to EMV

| Prototype | void importSignatureStatus(int status) |  |
| --- | --- | --- |
| Feature | Import signature result to EMV procedure. |  |
| Parameter | status [in] | process result, 0:Success, 1:Fail |
| Return | None |  |
| Comment | None |  |

3.7.1.20 Read transaction log

| Prototype | int readTransLog(int logType, List<String> infoOut) |  |
| --- | --- | --- |
| Feature | Read card transaction log |  |
| Parameter | logType [in] | log type,0:transaction log, 1: trap log |
|  | infoOut [out] | Buffer, store the read logs. |
| Return | 0: Success Other value: Fail |  |
| Comment |  |  |

3.7.1.21 Abort transact process

| Prototype | void abortTransactProcess() |  |
| --- | --- | --- |
| Feature | Abort the emv transact process |  |
| Parameter | [in] | None |
| Return | None |  |
| Comment | Note: If EMV process not started, call this method has no effect, if EMV process already started, call this method can interrupt the process and result in EMVListenerV2.onTransResult() called |  |

3.7.1.22 Import data exchange status to emv

| Prototype | void importDataExchangeStatus(int status) |  |
| --- | --- | --- |
| Feature | Import data exchange status to emv procedure. |  |
| Parameter | status[in] | Data exchange result, 0:Success, 1:Fail |
| Return | None |  |
| Comment | None |  |

3.7.1.23 Start transaction process(extended method)

| Prototype | void transactProcessEx(Bundle transData, EMVListenerV2 listener) |  |
| --- | --- | --- |
| Feature | Start EMV process, refer to transactProcess(EMVTransDataV2 transData, EMVListenerV2 listener) |  |
| Parameter | transData [in] | Emv transaction data, similar to EMVTransDataV2, this param can be set with following code: Bundle bundle = new Bundle(); bundle.putString("amount", amount); //transaction amount bundle.putString("transType", transType);//transaction type bundle.putInt("flowType", flowType);//flow type bundle.putInt("cardType", cardType);//card type bundle.putString("cashbackAmount",cashbackAmount);//cashback amount(MIR emv use this field) bundle.putInt("emvAuthLevel",level);//auth level,0-Normal,1-EC PBOC |
|  | listener[in] | Refer to EMVListenerV2 |
| Return | None |  |
| Comment | None |  |

3.7.1.24 Query electronic cash balance

| Prototype | int queryECBalance(Bundle bundle) |  |
| --- | --- | --- |
| Feature | Query electronic cash balance |  |
| Parameter | bundle[out] | Contains the following data: 9F51: String, Application currency code (Hex format) 9F79: long, Electronic cash balance |
| Return | None |  |
| Comment | None |  |

3.7.1.25 Add DRL LimitSet

| Prototype | int addDrlLimitSet(DrlV2 drl) |  |
| --- | --- | --- |
| Feature | Query electronic cash balance |  |
| Parameter | drl[in] | DRL LimitSet, refer to DrlV2 |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.7.1.26 Delete DRL LimitSet

| Prototype | int deleteDrlLimitSet(String programId) |  |
| --- | --- | --- |
| Feature | Delete one DRL LimitSet by programId, if want to delete all DRL LimitSets, set programId as null |  |
| Parameter | programId [in] | The program ID |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.7.1.27 Set terminal param(extended method)

| Prototype | void setTermParamEx(Bundle bundle) |  |
| --- | --- | --- |
| Feature | Set terminal param |  |
| Parameter | bundle[out] | Contains the following data: supportDRL: booean, Is support DRl function |
| Return | None |  |
| Comment | None |  |

3.7.1.28 Query All Aid or Capk list

| Prototype | int queryAidCapkList(int type, List<String> list) |  |
| --- | --- | --- |
| Feature | Query all Aid or Capk list in SDK |  |
| Parameter | type[in] | The query type, 0-query all Aids 1-query all capks |
|  | list[out] | The queried Aid or Capk list |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.7.1.29 EMV transact pre process

| Prototype | int transactPreProcess() |  |
| --- | --- | --- |
| Feature | EMV pre process |  |
| Parameter | [in] | None |
|  | [out] | None |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.7.1.30 Add RevocationList

| Prototype | int addRevocList(RevocListV2 revocList) |  |
| --- | --- | --- |
| Feature | Add or update a RevocationList |  |
| Parameter | revocList[in] | A revocationList, refer to Aidl constant. RevocListV2 |
|  | [out] | None |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.7.1.31 delete RevocationList

| Prototype | int deleteRevocList(RevocListV2 revocList) |  |
| --- | --- | --- |
| Feature | Delete one or all RevocationLists |  |
| Parameter | revocList[in] | The RevocationList to be delete, refer to Aidl constant. RevocListV2. If set this param as null, all RevocationLists will be deleted |
|  | [out] | None |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.7.1.32 Set system time

| Prototype | int sysSetTime(long timeStamp) |  |
| --- | --- | --- |
| Feature | Set system time |  |
| Parameter | timeStamp[in] | The timestamp to be set, unit: ms |
|  | [out] | None |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.7.1.33 Get system time

| Prototype | int sysGetTime(byte[] outData) |  |
| --- | --- | --- |
| Feature | Get system time, unit: s |  |
| Parameter | [in] | None |
|  | outData[out] | Buffer, store the got system time, this value is a yyyyMMddHHmmss string converted byte array, eg: hexStringToBytes(“20191130142020”) |
| Return | >=0: The valid data length in outData <0: Fail |  |
| Comment | None |  |

3.7.1.34 Clear data

| Prototype | int clearData(int opCode) |  |
| --- | --- | --- |
| Feature | Clear emv data |  |
| Parameter | opCode[in] | Operation code, 0-clear all data, 1-clear terminal data, 2-clear card data. Refer to Aidl contact. EMV clear data constant definition |
|  | [out] | None |
| Return | 0: Success Other value: Fail |  |
| Comment | None |  |

3.7.2 EMVListenerV2 callback methods

Note: This interface is the interface passed between AIDLs and must be implemented in accordance with the Aidl interface when the callback is passed.

3.7.2.1 Request App select

| Prototype | void onWaitAppSelect(List<EMVCandidateV2> candList, boolean isFirstSelect) |  |
| --- | --- | --- |
| Feature | If card has more than one app, request to select one app. Refer to EMVCandidateV2 |  |
| Parameter | candList [in] | The app list for selecting |
|  | isFirstSelect[in] | Is first time to select |
| Return | None |  |
| Comment |  |  |

3.7.2.2 Request app final select

| Prototype | void onAppFinalSelect (String tag9F06Value) |  |
| --- | --- | --- |
| Feature | Notify client the emv app final select |  |
| Parameter | tag9F06Value [in] | The value of tag 9F06 |
| Return | None |  |
| Comment |  |  |

3.7.2.3 Request Confirm Card number

| Prototype | void onConfirmCardNo(String cardNO) |  |
| --- | --- | --- |
| Feature | Request confirm card number. |  |
| Parameter | cardNO[in] | The card number to be confirmed |
| Return | None |  |
| Comment |  |  |

3.7.2.4 Request confirm certificate info

| Prototype | void onCertVerfiy(int certType, String certInfo) |  |
| --- | --- | --- |
| Feature | Request confirm certificate info if EMV process required |  |
| Parameter | certType[in] | Certificate type |
|  | certInfo[in] | Certificate info |
| Return | None |  |
| Comment | None |  |

3.7.2.5 Request input PIN

| Prototype | void onRequestShowPinPad(int pinType, int remainTimes) |  |
| --- | --- | --- |
| Feature | Request input PIN |  |
| Parameter | pinType[in] | PIN type, 0:online PIN，1: offline PIN |
|  | remainTimes[in] | The remain try times of offline PIN. If current is online PIN, this value is always -1, if it is the first time to try input PIN, this value is -1 too. |
| Return | None |  |
| Comment | None |  |

3.7.2.6 Requset signature

| Prototype | void onRequestSignature() |  |
| --- | --- | --- |
| Feature | Request user signature |  |
| Parameter | [in] | None |
| Return | None |  |
| Comment | None |  |

3.7.2.7 Requset online process

| Prototype | void onOnlineProc() |  |
| --- | --- | --- |
| Feature | Request online process |  |
| Parameter | [in] | None |
| Return | None |  |
| Comment | None |  |

3.7.2.8 Card data exchange complete

| Prototype | void onCardDataExchangeComplete() |  |
| --- | --- | --- |
| Feature | The kernel and card data exchange complete |  |
| Parameter | [in] | None |
| Return | None |  |
| Comment | None |  |

3.7.2.9 EMV procedure result

| Prototype | void onTransResult(int code, String desc) |  |
| --- | --- | --- |
| Feature | Send EMV procedure result to client. This method is mutex with onConfirmationCodeVerified(), that is, in an EMV procedure, only one of the 2 methods will be called. |  |
| Parameter | code [in] | The result code: 0-Success, 1-Offline approval, 2-Offline denial, <0-Error code. |
|  | desc[in] | The result message corresponding to result code. |
| Return | None |  |
| Comment | None |  |

3.7.2.10 Confirmation code verified

| Prototype | void onConfirmationCodeVerified() |  |
| --- | --- | --- |
| Feature | The confirmation code has been verified. This method is mutex with onTransResult(), that is, in an EMV procedure, only one of the 2 methods will be called. At this moment, only PayPass Contactless may call this method. |  |
| Parameter | [in] | None |
| Return | None |  |
| Comment | None |  |

3.7.2.11Request data exchage

| Prototype | void onRequestDataExchange(String cardNo) |  |
| --- | --- | --- |
| Feature | Request user exchange data with emv procedure(MIR use this method) |  |
| Parameter | cardNo[in] | The card number |
| Return | None |  |
| Comment | None |  |

- 4. Error Code Definition

| Error code | Error description |
| --- | --- |
| -100 | Incorrect number of parameters or length |
| -101 | Unsupported command |
| -1000 | System parameter error |
| -1001 | Feature not supported |
| -1002 | Initialization failed |
| -1003 | System time year error |
| -1004 | System time month error |
| -1005 | System time day error |
| -1006 | System time hour error |
| -1007 | System time minute error |
| -1008 | System time second error |
| -1009 | Hardware failure |
| -1010 | Buffer length error |
| -2000 | Card parameter error |
| -2001 | Have no card |
| -2002 | Multiple cards |
| -2100 | Magnetic card data decoding |
| -2500 | Module detection failed |
| -2501 | Drive core data structure error |
| -2502 | Module is not powered |
| -2503 | Carrier is not turned on |
| -2520 | Communication timeout |
| -2521 | Internal FIFO operation failed |
| -2522 | Communication frame error |
| -2523 | Communication character check error |
| -2524 | Communication conflict |
| -2525 | Signal in communication does not comply with the protocol |
| -2526 | CRC check error in communication |
| -2527 | M1 card password authentication error |
| -2528 | Mifare authentication parameters are incorrect |
| -2529 | Card exists |
| -2540 | The number of data in the A card communication response does not match the expected |
| -2541 | A card communication replies to the first character of the WUPA/REQA command is illegal |
| -2542 | Card number checksum error of A card communication response |
| -2543 | The first character of the card number answered by the A card communication is wrong. |
| -2544 | A card communication response ATS TL byte is illegal |
| -2545 | The A0 T0 byte of the A card communication response is illegal. |
| -2546 | The A1 TA1 byte of the ATS communication is illegal. |
| -2547 | The ATB TB1 byte of the A-card communication response is illegal. |
| -2548 | A card communication response ATS TC1 byte is illegal |
| -2550 | The number of data in the B card communication response does not match the expected |
| -2551 | The first character of the B card communication response WUBP/REQB command is not 0x50 |
| -2552 | The fourth bit of the protocol type byte in ATQB is not '0' |
| -2553 | The B-card communication responds to the difference in channel coding and setting in the ATTRIB command. |
| -2554 | B card communication response HLTB command response non 0x00 error |
| -2560 | Retransmitted to the limit if the correct reception is received |
| -2561 | Block type coding error |
| -2562 | I block PCB error or subsequent data length error |
| -2563 | PICC uses I block response link block |
| -2564 | Received I block serial number is incorrect |
| -2565 | R block PCB error or subsequent data length error |
| -2566 | PICC response NAK block |
| -2567 | The received R block serial number is incorrect. |
| -2568 | S block PCB error or subsequent data length error |
| -2569 | S block non-S-WTX request sent by PICC |
| -2570 | WTX parameter error requested (=0) |
| -2571 | Card return data exceeds FSD |
| -2800 | Verify error |
| -2801 | Communication timeout |
| -2802 | Module is not powered |
| -2803 | ATR error |
| -2804 | Communication error |
| -3000 | Security parameter error |
| -3001 | Root key error |
| -3002 | Security system is locked |
| -3003 | Security file read and write error |
| -3004 | Key index error |
| -3005 | Key verification error |
| -3006 | No PIN entry |
| -3007 | PIN input canceled |
| -3008 | PIN input timeout |
| -3009 | PIN input interval is too short |
| -3010 | KCV mode error |
| -3011 | KCV check error |
| -3012 | KCV ODD check error |
| -3013 | No matching key |
| -3014 | Error of key type |
| -3015 | Key length error |
| -3016 | Key exponent length error |
| -3017 | Destination key index error |
| -3018 | Source key index error |
| -3019 | Source key type error |
| -3020 | Group index error |
| -3022 | No KCV |
| -3023 | DUKPT overflow |
| -3024 | DUKPT key type error |
| -3025 | DUKPT KSN needs to add 1 |
| -3026 | Try to use the key outside the scope of the key. |
| -3027 | Incorrect use of the key, restricting only decrypted keys to encrypt data, such as calculate the Mac with the master key |
| -3028 | Feature not yet supported |
| -3029 | Feature key attribute does not match |
| -3030 | Not certified |
| -4000 | Transaction refuse |
| -4001 | Please use other interfaces |
| -4002 | Transaction terminated |
| -4005 | Final select data error |
| -4100 | Transaction terminated (command send and receive error) |
| -4101 | Transaction terminated (command receipt timeout) |
| -4102 | Transaction terminated (command receipt timeout) |
| -4103 | Transaction terminated (status code error) |
| -4104 | Transaction terminated (card locked) |
| -4105 | Transaction terminated (application locked) |
| -4106 | Transaction terminated (terminal not applied) |
| -4107 | Transaction terminated (applications with no support for the terminal and card) |
| -4108 | Transaction terminated (card return data error) |
| -4109 | Transaction terminated (card return data element duplicate) |
| -4110 | Transaction terminated (transaction not received) |
| -4111 | Transaction terminated (card expired) |
| -4112 | The list of preprocessing parameters is empty |
| -4113 | Transaction terminated (L1 read card timeout) |
| -4114 | Transaction terminated (L1 transmission error) |
| -4115 | Transaction terminated (L1 protocol error) |
| -4116 | Transaction terminated (L2 mandatory data error) |
| -4117 | Transaction terminated (L2 card authentication failed (offline data authentication failed)) |
| -4118 | Transaction terminated (L2 status word error) |
| -4119 | Transaction terminated (L2 data parsing failed) |
| -4120 | Transaction terminated (L2 transaction amount exceeds the contactless transaction limit) |
| -4121 | Transaction terminated (L2 card data error) |
| -4122 | Transaction terminated (L2 does not support magnetic stripe card mode) |
| -4123 | Transaction terminated (L2 card without PPSE) |
| -4124 | Transaction terminated (L2 PPSE processing error) |
| -4125 | Transaction terminated (L2 candidate list is empty) |
| -4126 | Transaction terminated (L2 IDS read error) |
| -4127 | Transaction terminated (L2 IDS write error) |
| -4128 | Transaction terminated (L2 IDS data error) |
| -4129 | Transaction terminated (L2 IDS has no matching AC) |
| -4130 | Transaction terminated (L2 terminal data error) |
| -4131 | Transaction terminated (L3 timeout) |
| -4132 | Transaction terminated (L3 cancel) |
| -4133 | Transaction terminated (L3 transaction amount does not exist) |
| -4134 | Transaction terminated (re-presentation of the card) |
| -4135 | Transaction terminated (using other cards (with Data Record)) |
| -4136 | Transaction terminated (using other cards) |
| -4137 | Transaction terminated (GPO response error) |
| -4138 | Transaction terminated (final selection of card data error) |
| -4139 | Transaction terminated (L3 no DET data) |
| -4140 | Kernel type is not supported |
| -4141 | Contactless transaction limit exceeded |
| -4142 | The amount is 0 |
| -4144 | Please use another interface (pre-processing failed) |
| -4500 | Invalid parameter |
| -4501 | Checksum error when downloading public key |
| -4502 | Terminal parameters do not exist. |
| -4503 | Error with terminal parameter data |
| -4504 | The transaction log does not exist |
| -4505 | Transaction log data error |
| -4506 | EMV data does not exist |
| -4507 | PBOC LOG format does not exist |
| -4825 | Two present card |
| -4854 | Complete command with empty |
| -4855 | Complete command with ODOL |
| -4856 | Complete command after reselect app |
| -4857 | Read record command after reselect app |
| -7001 | Printer error |
| -7002 | Low battery voltage |
| -7003 | Out of paper |
| -7004 | Temperature is too high |
| -7005 | Printer data error |
| -7006 | Invalid print parameters |
| -7007 | Device not open or device operation error |
| -7008 | Print buffer overflow |
| -8001 | Write data fail for tax control module |
| -8002 | Read data fail for tax control module |
| -10100 | Serial port closed |
| -10101 | Serial port overtime |
| -10102 | LRC check error |
| -10103 | SP Out of sequence |
| -10104 | SP is initializing |
| -10105 | SP is rebooting |
| -10106 | SP is reconnecting. |
| -10107 | SP is busy |
| -10108 | SP is sleep |
| -10200 | Read OS file package error |
| -10201 | SP is updating |
| -10202 | Connect SP failed |
| -10203 | Failed to open upgrade file |
| -10204 | Packet timeout |
| -10205 | Packet processing error |
| -10206 | Upgrade string is too long |
| -10207 | Upgrade unsuccessful |
| -10208 | Did not get the sdk version number of this machine |
| -10209 | The version is the same as the target upgrade version. |
| -10210 | Query default information failed |
| -10211 | Firmware version does not allow downgrade |
| -10212 | Upgrade cancelled |
| -10300 | Input parameter error |
| -10301 | The length of the response packet data area is illegal. |
| -10302 | Answer packet data parsing error |
| -10400 | The kernel has been rebooted |
| -20001 | Repeated call |
| -20002 | Firmware is being upgraded |
| -20003 | Parameter error |
| -20004 | Thread was aborted |
| -20005 | Firmware upgrade failed |
| -20006 | Firmware verification failed |
| -30001 | Failure to read card (unknown cause card failure, recommended to re-read the card operation) |
| -30002 | Unknown card type |
| -30003 | Failure of NFC check card |
| -30004 | Failure of IC check card |
| -30005 | Read card timeout |
| -30013 | This card is a chip card and can not fallback. |
| -30014 | Create candidate list timeout |
| -30015 | Card interaction failure |
| -30016 | Wrong card interaction parameters |
| -40002 | Key length error |
| -40003 | The check value error |
| -40004 | Store key fail |
| -40005 | Calculate MAC error |
| -40006 | Encryption error |
| -40007 | Return array data length error |
| -40008 | The MAC algorithm type not support |
| -40009 | Length of check value error |
| -40010 | Key index error |
| -40011 | Decrypt error |
| -40012 | Length of key error |
| -40013 | Get random error |
| -40014 | Key does not exist |
| -40016 | Verify signature fail |
| -40017 | Failed to get alarm information code |
| -40018 | Key partition has run out |
| -40019 | Inject BDK error |
| -50002 | Transaction preprocess fail |
| -50003 | Transaction process fail |
| -50004 | EMV kernel process fail |
| -50005 | PAN format error |
| -50006 | Call PINPAD fail |
| -50007 | None kernel data |
| -50008 | PINPAD parameter error |
| -50009 | EMV process not finish |
| -50010 | The transaction type not support |
| -50011 | Checking card information fail or timeout. |
| -50012 | CVM error |
| -50013 | Database operation fail |
| -50014 | No matching CAPK |
| -50015 | Save terminal parameter error |
| -50016 | No matching AID |
| -50017 | Check card fail |
| -50018 | Call interface order error |
| -50019 | Transaction data invalid |
| -50020 | PIN entry cancel |
| -50021 | PIN entry error |
| -50022 | The index of app select error |
| -50023 | Cert verify error |
| -50024 | Online process error |
| -50025 | App final select timeout |
| -50026 | App final select error |
| -50027 | Signature error |
| -50028 | Unknown CVM type |
| -60001 | Input PIN timeout |
| -60002 | Keyboard failed to activate password |
| -60003 | PinPadType type error (when the incoming keyboard type is not 1 and 2, the error is returned) |
| -60004 | Getting PinBlock failed |
| -60005 | PIN status query thread is interrupted |
| -70001 | Miss permisstion com.sunmi.perm.MSR |
| -70002 | Miss permission com.sunmi.perm.ICC |
| -70003 | Miss permission com.sunmi.perm. CONTACTLESS_CARD |
| -70004 | Miss permission com.sunmi.perm. PINPAD |
| -70005 | Miss permission com.sunmi.perm. SECURITY |
| -70006 | Miss permission com.sunmi.perm. LED |

- 5. Entity class

- 5.1 EmvTermParamV2 – Terminal parameter entity class

Class member variable declaration:

public String ifDsn = "3030303030393035"; // IFD sn public String terminalType = "22"; // Terminal type public String countryCode = "0156"; // Terminal country code public boolean forceOnline = false; // Merchant force online(1-online) public boolean getDataPIN = true; // Whether to read the number of retries before the password is detected? public boolean surportPSESel = true; // Support PSE select public boolean useTermAIPFlg = true; // Is risk management based on card AIP? public boolean termAIP = true; // Whether the terminal enforces risk management? public boolean bypassAllFlg; // After processing a PIN in byPass mode, whether other PINs are also handled in bypass mode? public boolean bypassPin = true; // Support bypass PIN? public boolean batchCapture; // Whether to batch capture data? public boolean ectSiFlg = true; // Does EC Terminal Support Indicator exist? public boolean ectSiVal = true; //Whether to support the electronic cash terminal support indicator? public boolean ectTlFlg = true; //Does EC Terminal Transaction Limit exist? public String ectTlVal = "100000"; // Electronic cash terminal transaction limit, unit cent public String capability = "E0F8C8"; // Terminal capability public String addCapability = "0300C00000";// Terminal extended capability public boolean scriptMode; // scriptMode public boolean adviceFlag = true; // adviceFlag public boolean isSupportSM = true; // Support SM? public boolean isSupportTransLog = true; // Support transaction LOG? public boolean isSupportMultiLang = true; // Support multiple language? public boolean isSupportExceptFile = true; // Support exception file? public boolean isSupportAccountSelect = true; // Support account select? public String TTQ = "26000080"; // Terminal transaction attribute (For contactless) public boolean IsReadLogInCard; // Is it an application selection process for reading in-card transaction records? private byte[] reserved = new byte[3]; // Reserved byte value must be 0

- 5.2 AidV2 -AID entity class

public byte[] aid; //AID public byte[] cvmLmt = new byte[6]; //ContactLess CVM Limit public byte[] termClssLmt = new byte[6]; //ContactLess Trans Limit public byte[] termClssOfflineFloorLmt = new byte[6]; //ContactLess Floor Limit public byte[] termOfflineFloorLmt = new byte[6]; //Terminal electronic cash transaction limit public byte selFlag; //ASI public byte targetPer;// Random target percentage public byte maxTargetPer; // Offset the maximum target percentage randomly selected public byte[] floorLimit; //Terminal Floor Limit public byte randTransSel; // Whether to make random trading choices public byte velocityCheck;// Whether to perform frequency detection public byte[] threshold = new byte[4];// Bias randomly selected thresholds public byte[] TACDenial = new byte[5];// TAC-Denial public byte[] TACOnline = new byte[5];// TAC-Online public byte[] TACDefault = new byte[5]; //TAC-Default public byte[] AcquierId = new byte[6];// Acquirer ID public byte[] dDOL; //Default DDOL public byte[] tDOL; // Default TDOL public byte[] version = new byte[2]; //Application version public byte rMDLen; // Risk management data length public byte[] riskManData = new byte[8]; // Risk management data public byte[] merchName = new byte[128]; //Merchant Name public byte[] merchCateCode = new byte[2];// Merchant type code public byte[] merchId = new byte[16]; //Merchant ID public byte[] termId = new byte[8]; //Teminal ID public byte[] referCurrCode = {0x01, 0x56}; //Reference currency code public byte referCurrExp; //Reference currency code exponent public byte[] referCurrCon = new byte[4]; // Conversion Coefficient of Reference Currency Code and Transaction currency Code(currently, this field is ignored) public byte clsStatusCheck; //Contactless status check public byte zeroCheck; //zero amount check

- 5.3 CapkV2 –CAPK entity class

public byte[] rid = new byte[5];// CAPK RID public byte index; //CAPK KeyID public byte hashInd;// CAPK hash algorithm indicator public byte arithInd; //CAPK RSA algorithm indicator public byte[] modul; // CAPK modulus public byte[] exponent; // CAPK Exponent public byte[] expDate = new byte[3];// CAPK Expiry Date(YYMMDD) public byte[] checkSum = new byte[20]; // CAPK CheckSum

- 5.4 EMVTransDataV2 - Transaction data entity class

public String amount; // Transaction amount(unit: cent)，must have parameter，cannot be null or "" , when amount="0" means check the balance. public String transType = "00"; // Transaction type default padding with "00". public int flowType = 01; // flow type，0x01:Standard flow；0x02:Simple flow；0x03 qPass public int cardType = 2; //Card type, 2:IC 4:NFC

- 5.5 EMVCandidateV2 – EMV Application Candidate

public short index;//Index,corresponding to the priority list public String aid;//Card Aid public String appPreName;/App prefer name public String appLabel;//App label public String issDiscrData;//Data of tag 'BF0C’ public byte priority;//Priority flag public String appName;//Local app name public byte kernelType;//The kernel type of contactless App

- 5.6 PinPadConfigV2 - Pinpad configuration entity class

public class Pinpadconfig implements Parcelable { private int pinpadType; // PinPad type. 0：Default Pinpad 1:Self-defined Pinpad private int pinType = 0; // Pin type (0: Online PIN，1:Offline PIN) private boolean isOrderNumKey = false; // true: Normal Pinpad; false: Random Pinpad private byte[] pan; // Ascii format to byte. eg. “123456”.getbytes("us ascii") private int pinkeyIndex; // Pin Key Index private int maxInput = 6; // Maximum password input (max 12 numbers) private int minInput = 0; // Minimum password input private int timeout = 60000; // Time out/millisecond private boolean isSupportbypass = true;//support bypasspin? private int pinblockFormat = 0; //pinblock format,Support the following formats： SEC_PIN_BLK_ISO_FMT0(0) SEC_PIN_BLK_ISO_FMT1(1) SEC_PIN_BLK_ISO_FMT2(2)：If the offline transaction defaults to this format, you do not need to set it. SEC_PIN_BLK_ISO_FMT3(3) //pinblock ISO-9564，Please refer to https://en.wikipedia.org/wiki/ISO_9564 private int algorithmType = 0; //Encrypted Pin algorithm type 0-3DES (returns 8 bytes), 1-SM4 (returns 16 bytes) private int keySystem = 0; // The key system to which Pik currently belongs is 0-SEC_MKSK, 1-SEC_DUKPT. Reference Appendix: Aidl Constant Key System Constant }

- 5.7 DrlV2 – DRL LimitSet entity class

public boolean isDefaultLmt = false; //Is default limitSet public boolean statusCheck = false; //Is enable status check public byte zeroCheck = 1; //Is enable zero amount check, 0-online,1-not allow,2-disable public byte[] programID; //Program ID public byte[] cvmLmt = new byte[6]; // ContactLess CVM Limit public byte[] termClssLmt = new byte[6]; // ContactLess Trans Limit public byte[] termClssFloorLmt = new byte[6];//Terminal Contactless floor limit public byte[] termFloorLmt = new byte[6];//Terminal floor limit public boolean cvmLmtActivate = true;//Is enable CVM limit check public boolean termClssLmtActivate = false; //Is enable terminal contactless limit check public byte termClssFloorLmtActivate = 1;//Is enable terminal contactless floor limit,0-disable,1-enable,2-enable but contactless floor limit not exist

- 5.8 RevocListV2– RevocationList entity class

public byte[] rid= new byte[5];// Application registration service provider ID public byte index;//Key index public byte[] sn= new byte[3];//Serial number public byte[] reserved = new byte[3];//Reserved bytes, all bytes should be 0

- 6. Access permission

- 6.1 Permission location

The appropriate permissions should be declared in the AndroidManifest file before using each interface.

- 6.2 Permission definition

6.2.1 Magnetic stripe card permission

<uses-permission android:name="com.sunmi.perm.MSR"/>

6.2.2 Contact IC card permission

<uses-permission android:name="com.sunmi.perm.ICC"/>

6.2.3 Contactless IC card permission

<uses-permission android:name="com.sunmi.perm.CONTACTLESS_CARD"/>

6.2.4 Pinpad permission

<uses-permission android:name="com.sunmi.perm.PINPAD"/>

6.2.5 Security permission

<uses-permission android:name="com.sunmi.perm.SECURITY"/>

6.2.6 LED permission

<uses-permission android:name="com.sunmi.perm.LED" />

6.2.7 Printer permission (not supported)

<uses-permission android:name="com.sunmi.perm.PRINTER" />

6.2.8 Serial port permission (not supported)

<uses-permission android:name="com.sunmi.perm.SERIAL"/>

6.2.9 Customer display permission (not supported)

<uses-permission android:name="com.sunmi.perm.CUSTOMER_DISPLAY"/>

6.2.10 ID card permission (not supported)

<uses-permission android:name="com.sunmi.perm.IDCard"/>

6.2.11 Money box permission (not supported)

<uses-permission android:name="com.sunmi.perm.MONEYBOX"/>

6.2.12 Finger print permission (not supported)

<uses-permission android:name="com.sunmi.perm.FINGERPRINT"/>

- 7. Appendix

- 7.1 Aidl constants class（com.sunmi.pay.hardware.aidl.AidlConstants）

7.1.1 Card type constant definition

// Magnetic public static final int MAGNETIC= 1<<0; // IC public static final int IC = 1<<1; // RFC public static final int NFC = 1<<2; // Mifare public static final int MIFARE= 1<<3; // PSAM,slot 0 public static final int PSAM0= 1<<4; // Felica public static final int FELICA= 1<<5; // SAM1 public static final int SAM1= 1<<6; // Mifare plus public static final int MIFARE_PLUS= 1<<7; // Mifare desfire public static final int MIFARE_DESFIRE= 1<<8; // AT24C01 public static final int AT24C01= 1<<9; // AT24C02 public static final int AT24C02= 1<<10; // AT24C04 public static final int AT24C04= 1<<11; // AT24C08 public static final int AT24C08= 1<<12; // AT24C16 public static final int AT24C16= 1<<13; // AT24C32 public static final int AT24C32= 1<<14; // AT24C64 public static final int AT24C64= 1<<15; // AT24C128 public static final int AT24C128= 1<<16; // AT24C256 public static final int AT24C256= 1<<17; // AT24C512 public static final int AT24C512= 1<<18; // SLE4442 public static final int SLE4442= 1<<19; // SLE4428 public static final int SLE4428= 1<<20; // AT88SC1608 public static final int AT88SC1608= 1<<21; // CTX512B public static final int CTX512B= 1<<22;

7.1.2 Key type constant definition

// KEK (Key encrypt key) public final static int KEY_TYPE_KEK = 0x01; // TMK (Terminal master key) public final static int KEY_TYPE_TMK = 0x02; // PIK (PIN key) public final static int KEY_TYPE_PIK = 0x03; // MAK (Mac key) public final static int KEY_TYPE_MAK = 0x04; // TDK (Track data key) public final static int KEY_TYPE_TDK = 0x05; // Reserved public final static int KEY_TYPE_REC = 0x06; // Dupkt BDK (Base derived key) public static final int KEY_TYPE_DUPKT_BDK = 0x07; // Dukpt IPEK (Initial PIN encryption key) public static final int KEY_TYPE_DUPKT_IPEK = 0x08; // TR31 KBPK (Key block protection key) public static final int KEY_TYPE_KBPK = 0x09;

7.1.3 Key algorithm type constant definition

// Encryption type: 3DES or DES public final static int KEY_ALG_TYPE_3DES = 0x01; // Encryption type: AES public final static int KEY_ALG_TYPE_AES = 0x02; // Encryption type: SM4 public final static int KEY_ALG_TYPE_SM4 = 0x03;

7.1.4 MAC algorithm constant definition

// ISO9797-1-ALG1 public static final int MAC_ALG_ISO_9797_1_MAC_ALG1 = 1001; // ISO9797-1-ALG3 public static final int MAC_ALG_ISO_9797_1_MAC_ALG3 = 1003; // Same as ISO9797-1-ALG3 public static final int MAC_ALG_ISO_16609_MAC_ALG1 = 2000; // ECB algorithm. public static final int MAC_ALG_FAST_MODE = 3000; // X9_19 algorithm mac public static final int MAC_ALG_X9_19 = 3001; // CBC UnionPay algorithm public static final int MAC_ALG_CBC = 3002; // FAST_MODE internation standard mac public static final int MAC_ALG_FAST_MODE_INTERNATIONAL = 30000; // CBC international standard mac public static final int MAC_ALG_CBC_INTERNATIONAL = 30001;

7.1.5 Certificate type constant definition

// ID card public static final int IDCARD = 536911872; // Certificate of officers public static final int ARMYCARD = 536911873; // Passport public static final int PASSPORT = 536911874; // Arrival card public static final int ARRIVALCARD = 536911875; // Temporary ID card public static final int TEMPIDCARD = 536911876; // Other certificate public static final int OTHERCARD = 536911877;

7.1.6 EMV module related constant definition

// Force online public static final int FORCE_ONLINE = 0; // No online public static final int NO_ONLINE = 1; // CAPK and AID are not exist. public static final int EXIST_ALL_NOT = -1; // CAPK and AID are exist. public static final int EXIST_ALL= 0; // CAPK is not exist public static final int EXIST_CAPK_NOT = 1; // AID is not exist. public static final int EXIST_AID_NOT = 2; // Transaction complete. public static final int EMV_RESULT_FINISHED = 0x9000; // Transaction abort. public static final int EMV_RESULT_TERMINATION = 0x9001; // Get PINBLOCK failure. public static final int EMV_ERROR_PINBLOCK = 0x9002; // Transaction is not supported. public static final int EMV_UNSUPPORTED_TRANS = 0X9003;

7.1.7 EMV FlowType constant definition

// Sandard authorization flow public static final int FLOW_TYPE_1 = 0x01; // Simple flow, finished as soon as read card number public static final int FLOW_TYPE_2 = 0x02; // QPASS flow-Skip input PIN in contactless transaction public static final int FLOW_TYPE_3 = 0x03; // Contactless speedup flow public static final int FLOW_TYPE_4 = 0x04;

7.1.8 EMV clean data constant definition

// Clear all data public static final int OP_CLEAR_DATA_ALL = 0; // Clear terminal data public static final int OP_CLEAR_DATA_TERMINAL = 1; // Clear card data public static final int OP_CLEAR_DATA_CARD = 2;

7.1.9 AID function behavior constant definition

// Add or update a AID. public static final int ACTION_AID_ADD = 0x00; // Delete all AIDs. public static final int ACTION_AID_DEL = 0x01;

7.1.10 CAPK function behavior constant definition

// Add or update a CAPK. public static final int ACTION_CAPK_ADD = 0x00; // Delete all CAPKs. public static final int ACTION_CAPK_DEL = 0x01;

7.1.11 System parameter constant definition

// SDK version public static final String SDK_VERSION = "SDKVersion"; // Hardware version public static final String HARDWARE_VERSION = "HardwareVersion"; // Firmware version public static final String FIRMWARE_VERSION = "FirmwareVersion"; // SN public static final String SN = "SN"; // PN public static final String PN = "PN"; // TUSN public static final String TUSN = "TUSN"; // Device code(eg.W6900(4G)) public static final String DEVICE_CODE = "DeviceCode"; // Device model(eg.P1N) public static final String DEVICE_MODEL = "DeviceModel"; // Reserved. (value is Json format- extensible) public static final String RESERVED = "Reserved"; // PIN input mode public static final String PINPAD_MODE = "PinPadMode"; // EMV version public static final String EMV_VERSION = "EMVVersion"; // Paypass version public static final String PAYPASS_VERSION = "PaypassVersion"; // Paywave version public static final String PAYWAVE_VERSION = "PaywaveVersion"; // QPBOC version public static final String QPBOC_VERSION = "QPBOCVersion"; // Entry version public static final String ENTRY_VERSION = "EntryVersion"; // Mir version public static final String MIR_VERSION = "MirVersion"; // JCB version public static final String JCB_VERSION = "JCBVersion"; // Pago version public static final String PAGO_VERSION = "PAGOVersion"; // PURE version public static final String JCB_VERSION = "PUREVersion"; // AE version public static final String PAGO_VERSION = "AEVersion"; // EMV kernel checksum public static final String EMV_KERNEL_CHECKSUM = "EmvKernelCheckSum";

7.1.12 Led constant definition

// Red light public static final int RED_LIGHT = 1; // Green light public static final int GREEN_LIGHT = 2; // Yellow light public static final int YELLOW_LIGHT = 3; // Blue light public static final int BLUE_LIGHT = 4;

7.1.13 Encryption mode constant definition

public static final int DATA_MODE_ECB = 0; public static final int DATA_MODE_CBC = 1; public static final int DATA_MODE_OFB = 2; public static final int DATA_MODE_CFB = 3;

7.1.14 Key system type constant definition

public static final int SEC_MKSK = 0x00; public static final int SEC_DUKPT = 0x01;

7.1.15 Dukpt key select constant definition

// DUKPT PIN key public static final int DUKPT_KEY_SELECT_KEY_PIN = 0; // DUKPT Request&Reponse MAC key public static final int DUKPT_KEY_SELECT_KEY_MAC_BOTH = 1; // DUKPT Response MAC key public static final int DUKPT_KEY_SELECT_KEY_MAC_RSP = 2; // DUKPT Request&Response data key public static final int DUKPT_KEY_SELECT_KEY_DATA_BOTH = 3; // DUKPT Response data key public static final int DUKPT_KEY_SELECT_KEY_DATA_RSP = 4;

7.1.16 RSA transformation constant definition

public static final String RSA_TRANSFORMATION_1 = "RSA/None/NoPadding"; public static final String RSA_TRANSFORMATION_2 = "RSA/None/PKCS1Padding"; public static final String RSA_TRANSFORMATION_3 = "RSA/ECB/NoPadding"; public static final String RSA_TRANSFORMATION_4 = "RSA/ECB/PKCS1Padding";

7.1.17 RSA signature algorithm constant definition

public static final String RSA_SIGN_ALG_1 = "NONEwithRSA"; public static final String RSA_SIGN_ALG_2 = "MD5withRSA"; public static final String RSA_SIGN_ALG_3 = "SHA1withRSA"; public static final String RSA_SIGN_ALG_4 = "SHA256withRSA"; public static final String RSA_SIGN_ALG_5 = "SHA512withRSA";

7.1.18 PinPad mode constant definition

// Normal public static final String MODE_NORMAL = "Normal"; // MeiTuan public static final String MODE_MEITUAN = "MeiTuan";

- 7.2 PAN data interception

Starting from the second digit on the right side of the card number (Field 2), the 12 bit is taken to the left as the PAN to participate in PIN encryption and decryption.

For example, the card number is 6225882145611077.

PAN data after interception 588214561107

Convert to byte[] array "588214561107".GetBytes ("US-ASCII");

All the PAN data that needs to be passed in the SDK is intercepted in this way, and if the pinblock result is incorrect, check the incoming PAN data in accordance with the above rules if the pinKey is correct

- 7.3 EMV trans flow chart